================================================================

====================== ORTEP Upgrades ==========================

================================================================

[1] Formats Of Input Data File:

Crystal symmetry  card = SENTINEL, S, T

Atomic positional card = LABEL(A8), POSITION, COORDINATE-TYPE, ID

where ID is non-zero integer identifying the species of input atoms.

Certain ORTEP commands have been expanded to handle the atom ID as well

as atom number run in vector search code (see Expanded ORTEP Commands).

Notice that the atom label has been expanded to 8 characters.

All input data are free of field formats. Both decimals and integers can

be used for any data fields. Space(s) and comma are valid field separators.

Blank lines and TABs are allowed. Column line-up of fields are not necessary.

Additional comment lines can be entered with a character "C" in the 1st column

or any line starting with a "C" in the 1st column is ignored. Extra one card

for lattice centering symmetry may follow if SENTINEL=2 in the last symmetry

card:

Format  : X1 Y1 Z1 X2 Y2 Z2 X3 Y3 Z3

Examples: .5 .5 .5  0  0  0  0  0  0  ==> I-center

.5 .5  0 .5  0 .5  0 .5 .5  ==> F-center

A negative sentinel signifies a centrosymmetric lattice.

[2] Implementation of a Digit Flag for Atom Designator Code (ADC):

ORTEP uses a REAL*4 array to store input and sorted atom list in ADC.

The REAL*4 array does not have the precision to store an ADC of value

greater than 16755501. As a result, the input atom table is limited to

max 167 atoms (the original dimension is 100). There are two ways to

expand the input atom table over 167 in order to draw a large molecule.

One is to make the atom array REAL*8, which was proven to be too much

to accomplish. The second is to reduce the symmetry digits (the last 2

digits) of an ADC from 2 to 1 (e.g. from 16755501 to 1675551). In doing

so, the max number of symmetry elements can be entered is reduced from

99 to 9 and the max number of input atoms (not the max number of atoms

that can be plotted) is expanded to about 1670. The later has been

implemented here as an option in the cell parameter card as follows:

a, b, c, alpha, beta, gamma, digit_flag

Digit_flag=0 signifies 2-symmetry-digit ADCs in user input file.

Digit_flag=1 signifies 1-symmetry-digit ADCs in user input file.

Program is dimensioned to handle 999 input atoms and 999 sorted atoms.

[3] Expanded ORTEP Commands:

1) 999 :

command syntax: lookahead  999  pen-number

where the valid pen numbers are from 1 to 8 corresponding to the

eight different colors/pens on graphics terminals/pen plotters.

This command allows for changing colors in drawing atoms or bonds

or labels. Issue the desired 999 command before the drawing commands.

2) Vector Search Code (VSC):

511 and 700, 800 series commmands have been expanded to handle the

atom ID in place of VSC. If the 2nd atom number of an atom number

run in the VSC is zero, the 1st atom number is taken as atom ID.

Atom ID designation and atom number run can be mixed if desired

in a single VSC. The introduction of atom ID allows for plotting

of a desired group of atoms or bondings without sorting the input

atom list.

3) In plotting the atom labels, the labels are first stripped

of any leading and trailing blanks and then centered based

on the string length left. Therefore atom labels as entered

in input atom list do not have to center within the reserved

8 column spaces. ORTEP 700 series commands have been expanded

to plot shorter atom labels. This is done by resorting to an

8-digit code for the 8 columns of atom labels. The 8-digit code

is read from parameter #12 (P12) following the atom number run

of 700 series commands. If P12 = 0.0, the whole input atom label

will be plotted if enabled. To plot a single column or a few of

the 8 columns of the labels, enter P12 as e.g. 11000000, 1000,

11110, 111000, where digit 1 signifies the column of the label

characters to be plotted.

In addition, parameter #13 (P13) following P12 (as described above)

is also implemented as an optional pattern code for area fill/shading

in drawing the ellipsoids. The valid pattern code values for P13 are

from 1 to 17 corresponding to 17 different shading patterns. Naturally

pattern #1 is solid fill. If enabled, only non-overlapping ellipsoids

can be filled if the pattern code is positive. To fill overlapping

atoms as well, enter a negative pattern code.

[4] Interactive On-Line Graphics Commands:

1) ROTATE: Perform on-line incremental non-cumulative 3-d rotations.

On-line help is available for how to use it.

2)  %    : Rescale the percent probability ellipsoids.

On-line help is available for how to use it.

3) LAYOUT: Implement grafit's 2-d interactive layout function with

the parameter file I/O feature.

On-line help is available for how to use it.