$! File format documentation.  @ this file to split it.

$!

$CREATE ALAR.DOC

C ALAR -- ALFRED ASCII RAW DATA FILE FORMAT:

C	Record 1: Title (A80)

C	Record 2: Date (A9), Time (A8)

C	Record 3: Start angle, End angle, Angle increment, Count time (4F10.4)

C	Rest of file: Counts (10I8 or 10F8.1)

C

$CREATE ALBR.DOC

C ALBR -- ALFRED BINARY RAW DATA FILE FORMAT:

C	Record 1: Title (80 bytes)

C	Record 2: Date (9 bytes), Time (8 bytes)

C	Record 3: Start angle, End angle, Angle increment, Count time (real*4)

C	Rest of file: Counts per second, (10 per line, real*4)

C

$CREATE RIER.DOC

C RIER -- RIETVELD PROGRAM RAW DATA FILE FORMAT:

C	Record 1: Start angle, Angle increment, End angle (3F8.4)

C	Rest of file: Counts per second, (8(I7,X))

C

$CREATE DI5R.DOC

C DI5R -- DIFFRAC 5 RAW DATA FILE FORMAT:

C	File is binary direct access with fixed length 4 byte records.

C	Kermit transfers in binary mode will scramble it to 510 byte records.

C	Records 1- 4: (16 bytes) Sample ID

C	Records 5- 7:  (9 bytes) Date measured

C	Record  8:  Not used.  Everything past this record is REAL*4.

C	Record  9:  Start angle

C	Record 10:  End angle

C	Record 11:  Angle increment

C	Record 12:  Count time

C	Record 13-end: Counts

C

$CREATE D11R.DOC

C D11R -- DIFFRAC 11 RAW DATA FILE FORMAT:

C	File is binary direct access with fixed length 4 byte records.

C	Kermit transfers in binary mode will scramble it to 510 byte records.

C	Records  1- 4: (16 bytes) Sample ID.

C	Records  5- 7: (9 bytes)  Date measured

C	Records  8- 9: (8 bytes)  Time measured

C	Records 10-12: (12 bytes) Operator's name (copied from filloc.dat)

C	Records 13-29:	Copied from D500nn.DAT(1-17) with change to rec 19 (7)

C	  Record 13:   (4 bytes) Anode material

C	  Record 14:   (real*4)  Wavelength

C	  Record 15:   (real*4)  Delta lambda

C	  Record 16:   (2 int*2) Kilovolts and Milliamps

C	  Record 17:   (real*4)  Detector slit

C	  Record 18:             Apeture.  Negative or 0 means fixed.

C	Records 19:    (2 int*2) Detector (2*SC+Monochrom);

C    &			Meas mode (0=unknown, 1=stepscan, 2=scan)

C	  Records 20-24: (5 real*4) Calibration coefficients for 2-theta angles

C	  Records 25-29: (5 real*4) Calibration coefficients for intensity (PSD)

C	Records 30:    (2 int*2) Number of ranges;

C    &			 Operation code (2*delay time+Theta fixed. 0=norm)

C	   The following are REAL*4 and repeat for each range

C	Records 31-34	SANG,EANG,AINC,CTIME

C	Records 35	Theta fixed or delay time.

C	Records 36-...  Counts for first range.

C

$CREATE D11D.DOC

C D11D -- DIFFRAC 11 D & I FILE FORMAT

C	File is binary direct access with fixed length 4 byte records.

C	Kermit transfers in binary mode will scramble it to 510 byte records.

C	Records  1- 4: (16 bytes) Sample ID.

C	Records  5- 7: (9 bytes)  Date measured

C	Records  8- 9: (8 bytes)  Time measured

C	Records 10-12: (12 bytes) Operator's name (copied from filloc.dat)

C	Records 13-16: (4 real*4) Measuring parameters: SANG, EANG, AINC, CTIM

C	Record  17     reserved.

C	Record  18:   (4 bytes) Anode material

C	Record  19:   (real*4)  Wavelength

C	Record  20:	(2 bytes) Program that file: A=ADR, I=IDR;

C    &			(2 bytes) I=integral intensities, N=net intensities

C	Record  21:	(2 int*2)npeak; nresidual peaks

C	Record  22:	Program which stored residual intens: C=COMSEA, J=JCPDS

C	Record  23:	      Apeture (rec 18 of .RAW file)

C	Record  24:	?

C	Record  25:	(2 int*2) 1=unitcell stored by apple; 1=hkl stored

C	Records 26-27:	(6 bytes) crystal system; AND (2 bytes) Bravais lattice

C	Records 28-30:	(real*4) A,B,C of unit cell

C	Record  31:	(real*4) cell volume

C	Records 32-34:	(3 real*4) angles alpha, beta, gamma

C	Record  35-38:	reserved

C	    The rest repeats for each peak.

C	Records 39-40:	(4 int*2) h,k,l,0

C	Record  41:	(real*4)  D-value

C	Record  42:	(real*4)  Intensity

C	Record  43:	(real*4)  Full width at half maximum

C	Record  44:	(real*4)  Residual intensity

C

$EXIT