************************************************************************

DATAPP                                                         SCOPE 3.2

DATAPT                                                        JULY, 1971

S.U.N.Y./BUFFALO CRYSTALLOGRAPHIC COMPUTER PROGRAMS

************************************************************************

NOTE: REPLACEABLE ROUTINES IMP AND SCALE HAVE BEEN CHANGED TO IMPD

AND SCALED RESPECTIVELY.  THESE ARE PERMANENT.

CORE:

DATAPP:     OCTAL:  47000 + 4*N(1)*N(2)*N(3)

WHERE N(1), N(2), N(3) ARE THE NUMBER OF GAUSSIAN GRID

POINTS.

DATAPT:     60 K OCTAL  (FIXED VALUE)

PROGRAM CARD:  PROGRAM DATAPP (INPUT,OUTPUT,TAPE3,TAPE4,TAPE5=INPUT,

PUNCH,TAPE6=OUTPUT,TAPE1=TAPE3,TAPE2)

PURPOSE:

THIS IS THE SECOND STAGE IN PROCESSING OF X-RAY OR NEUTRON

FOUR-CIRCLE DIFFRACTOMETER DATA.  LORENTZ-POLARIZATION FACTORS ARE

APPLIED, ABSORPTION CORRECTIONS ARE MADE, AND THE NECESSARY QUAN-

TITIES FOR MAKING EXTINCTION CORRECTIONS IN THE ZACHARIASEN APPROX-

IMATION ARE CALCULATED (ZACHARIASEN, ACTA CRYST., 23, 558 (1967),

COPPENS AND HAMILTON, ACTA CRYST., A26, 71 (1970)).  FOR DETAILS ON

THE ABSORPTION CORRECTION, SEE ACTA CRYST., 18, 1035 (1965).

PROGRAM DATAPP CALCULATES THE ABSORPTION FACTORS BY NUMERICAL

GAUSSIAN INTEGRATION, WHILE PROGRAM DATAPT CALCULATES THEM BY THE

ANALYTICAL METHOD OF DE MEULENAER AND TOMPA (ACTA CRYST., 19, 1014,

(1965) ).  THE PROGRAMS ARE SO DESIGNED THAT THE INPUT DECK AND

ROUTINES MAY BE SUBMITTED AS INPUT TO EITHER PROGRAM.

TIME REQUIRED:

(THIS PROGRAM SHOULD ALWAYS BE TEST RUN ON ABOUT 20 DATA FOR

PURPOSES OF TIMING AND VERIFYING ALL ANSWERS ARE SATISFACTORY.)

DATAPP:

TWO EXAMPLES MAY SERVE AS A GUIDE:

1) NO ABSORPTION:  1302 REFLECTIONS IN 18.3 SEC CP TIME

2) 144 POINT GRID AND 6 PLANES:  2436 REFLECTIONS IN 370

SEC. CP TIME

DATAPP  2

DATAPT:

VERY FAST, EVEN WHEN ABSORPTION FACTORS ARE BEING CALCULATED.

APPROXIMATELY 1.0 CP SEC/REFLECTION.

TAPES:

TAPE 3 INPUT  OPTIONAL

TAPE 4 OUTPUT OPTIONAL  (BIN)

TAPE 2 OUTPUT OPTIONAL  (BCD)

INPUT:

INPUT TO BOTH PROGRAMS IS THE SAME EXCEPT WHERE NOTED.

1.  (1X79H)  TITLE.  PUNCH IN COLUMNS 2-80.

2.  (7F9.6)  CELL CONSTANTS.

ANY OF THE FOLLOWING MAY BE USED (A VALUE ` 1. IS CONSIDERED TO

BE A COSINE):

COLS.    1-9      A*          A*         A         A

10-18      B*          B*         B         B

19-27      C*          C*         C         C

28-36  COS(ALPHA*)   ALPHA*   COS(ALPHA)  ALPHA

37-45   COS(BETA*)    BETA*    COS(BETA)   BETA

46-54  COS(GAMMA*)   GAMMA*   COS(GAMMA)  GAMMA

FOR ANY OF THE FOUR COMBINATIONS ABOVE:

55-63  WAVELENGTH, IN ANGSTROMS.

3.  ORIENTATION MATRIX

OPTION 1.  (9F8.0)  ((WN(I,J),J=1,3),I=1,3)

WHERE @WN(I,J)? IS THE ORIENTATION MATRIX

DEFINED BY COPPENS AND BEDNOWITZ AND IS PRINTED

OUT BY THE ROUTINES XRAY, MAIN, AND SATT AT THE

HFBR.

SEE APPENDIX, PAGE 11.

ALPHA*(TRANSPOSE) = AG(TRANSPOSE).WN

DATAPP  3

IF THE TRANSFORMATION MATRIX U IS DEFINED AS IN

THE HAMILTON INTERNATIONAL TABLES ARTICLE:

A* = U.AG,

WHERE A* ARE THE RECIPROCAL CELL COORDINATES,

AND AG IS AN ORTHOGONAL SYSTEM FIXED IN THE

GONIOMETER HEAD, THEN THE WN MATRIX HERE IS

OBTAINED FROM U BY DIVIDING THE ROWS OF U BY

A*, B*, C*, RESPECTIVELY AND TAKING THE TRANS-

POSE.  IF ABSORPTION AND EXTINCTION CORRECTION

ARE UNDESIRED, THIS MATRIX IS IRRELEVANT.  WHEN

SUPPLIED, BE SURE THAT NORMALIZATION OF COLUMNS

TO UNITY IS EXACT.

OPTION 2. (PREFERRED OPTION)

U MATRIX TO BE CALCULATED FROM SETTING ANGLES

FOR 3 REFLECTIONS.

CARD 3.1  BLANK

CARD 3.2  (4F9.4)

COL.    1-9  PHIZRO:

10-18   OMZRO:  ANGLE

19-27 TWOTHZO:  ZEROS

28-36  CHIZRO:

CARD 3.3  (3F9.0,4F9.4)

COL.    1-9       H

10-18       K

19-27       L

28-36    PHI:

37-45     OM:  OBSERVED SETTING

46-54  TWOTH:  ANGLES FOR

55-63    CHI:  REFLECTION 1

CARD 3.4  SAME AS 3.3, FOR REFLECTION 2.

CARD 3.5  SAME AS 3.3, FOR REFLECTION 3.

4.  (16I5)  CONTROL INTEGERS (ICN(I),I=1,16)

COLS.    1-5  =  0 NO ABSORPTION CORRECTION.

=  1 ABSORPTION CORRECTION:  CRYSTAL BOUNDARY

PLANES DESCRIBED BY H, K, L INDICES AND

D IN CM.  (PREFERRED OPTION).

=  2 ABSORPTION CORRECTION:  CRYSTAL BOUNDARY

PLANES DESCRIBED BY OBSERVED SETTING

ANGLES NECESSARY TO BRING BOUNDING PLANE

INTO THE DIFFRACTING CONDITION, AND D IN

CM.

DATAPP  4

=  3 ABSORPTION CORRECTION:  CRYSTAL BOUNDARY

PLANES DESCRIBED BY EQUATION OF PLANES

IN SEATTLE COORDINATE SYSTEM, A, B, C,

AND D IN MM.

SEE SECTION ON BOUNDARY PLANES,

CARDS 7, PAGE 6.

6-10  =  0 NO QUANTITIES ARE CALCULATED FOR EXTINC-

TION.

= +1 QUANTITIES NECESSARY FOR EXTINCTION

CORRECTION IN LINUSU (S.U.N.Y./BUFFALO

LEAST-SQUARES PROGRAM) ARE CALCULATED.

(THIS OPTION DEMANDS CRYSTAL BOUNDARY

PLANES; SEE CARDS 7, PAGE 6).

= -1 VECTOR COMPONENTS BUT NOT TBAR ARE

CALCULATED FOR EXTINCTION.

(APPLICABLE IF ICN(1) = 0)

11-15  =  0 DO NOT CALL SCALE SUBROUTINE.

1 A SUBROUTINE SCALE IS CALLED WHICH

MAY BE USED TO SCALE DATA RELATIVE

TO STANDARDS COLLECTED DURING DATA

COLLECTION PROCEDURE.

2 APPLY A CUBIC POLYNOMIAL CORRECTION

TO DATA BASED ON STANDARD REFLECTIONS

A'LA ED MCGANDY

16-20     DATAPP ONLY.  DATAPT IGNORES THIS OPTION.

=  0 NORMALLY

=  1 CALCULATE DISTANCE BETWEEN GIVEN PAIRS

OF VERTICES.  (SEE CARDS 8)  VERTEX

CARDS FOLLOW CRYSTAL BOUNDARY PLANE

CARDS.

21-25  =  0 FOR X-RAY.

=  1 FOR NEUTRON DATA.

THIS INTEGER SPECIFIES FORM OF POLARI-

ZATION FACTOR.

26-30       DETERMINE STATUS OF BCD OUTPUT.

=  0 NO PUNCHED CARD OUTPUT OF REFLECTIONS,

NO BCD TAPE.

=  1 FOR PUNCHED CARD OUTPUT IN FORMAT SUIT-

ABLE FOR INPUT TO LINUSU OR DSORTH.

=  2 FOR OUTPUT ON BCD TAPE 2 IN FORMAT SUIT-

ABLE FOR INPUT TO LINUS OR DSORTH.  NO

UPDATING PERMITTED.

3 PUNCH AND WRITE TAPE 2

31-35       DETERMINES STATUS OF BINARY TAPE 4 FOR

LEAST-SQUARES INPUT.

= -1 FOR NO TAPE 4 OUTPUT.

DATAPP 5

=  0 FOR NEW TAPE 4 OUTPUT.

=  1 FOR AN UPDATED TAPE 4.  OLD TAPE IS READ

UNTIL END-OF-FILE; BACKSPACE OVER

END-OF-FILE AND ISENT = 1 RECORD, START

WRITING AND PLACE NEW ISENT = 1 RECORD

AND END-OF-FILE AT END OF NEW DATA.

36-40  =  0 FOR CARD INPUT AS DESCRIBED BELOW.

(USER MUST SUPPLY *IMP* ROUTINE)

SEE REPLACEABLE ROUTINES.

=  1 FOR INPUT ON TAPE3 AS DESCRIBED BELOW.

(USER MUST SUPPLY *IMP* ROUTINE)

SEE REPLACEABLE ROUTINES.

41-45  =  0 NORMALLY.

=  N FOR SETTING ANGLES CHECK.  IF ICN(9) = N

THIS CARD SHOULD BE FOLLOWED BY N CARDS

OF FORMAT (3F3.0,F9.3) WITH H, K, L AND

THE TRUE (ZERO CORRECTED) VALUE OF

OMEGA.  THE PROGRAM WILL CALCULATE AND

PRINT THE SETTING ANGLES FOR THESE RE-

FLECTIONS.

50  =  0 ALWAYS.

55  =  1 LAST DIGIT OF INTENSITY HANDLED

BY USER IN SUBROUTINE IMP

=  2 FOR STEP SCAN DATA FROM PROG. FILE

THE REMAINING FIELDS ON CARD 4:  ICN(I), I = 11-16 ARE

READ IN AND ARE AVAILABLE TO THE USER-SUPPLIED SUBROUTINES

FOR WHATEVER PURPOSE.

5.  OPTIONAL.  (3F3.0,F9.3)  H, K, L, OMEGA

ICN(9) CARDS FOR ANGLES CHECK.

COLS. 1-3   H

4-6   K

7-9   L

10-19   OMEGA(TRUE)

6.  OPTIONAL.  (4I5)  (NGAUSS(I),I=1,3),NFACES

SUPPLY ONLY IF ICN(1) IS > 0, OR ICN(2) > 0.

DATAPP:

SPECIFICATIONS FOR GRID FOR ABSORPTION OR EXTINCTION

CORRECTION.  A GAUSSIAN INTEGRATION FORMULA IS USED.  THE

FIRST THREE FIELDS DEFINE THE NUMBER OF GRID POINTS IN

THE DIRECTIONS A,B,C.  THE PRODUCT OF THESE THREE MUST BE

LESS THAN THE SECOND DIMENSION OF GRID IN THE MAIN PROGRAM.

THE VERSION OF THE PROGRAM AVAILABLE ON THE CRYSTAL SYSTEM

DATAPP  6

IS COMPILED FOR THIS DIMENSION EQUAL TO 5000.

COLS.    1-5  NUMBER OF GAUSSIAN GRID POINTS :

ALONG A. : MUST BE EVEN

6-10  NUMBER OF GAUSSIAN GRID POINTS : NUMBERS 2-32

ALONG B. :

11-15  NUMBER OF GAUSSIAN GRID POINTS : MUST BE EVEN

ALONG C. : NUMBERS 2-32

16-20  NUMBER OF BOUNDARY PLANES.

DATAPT:

COLS.  1-15  ARE IGNORED

16-20  ARE ABSOLUTELY NECESSARY

7.  OPTIONAL.  (4F10.6)  (SUPPLY IF CARD 6. SUPPLIED)

ONE CARD FOR EACH BOUNDARY PLANE, OPTION DEPENDING UPON

THE VALUE OF ICN(1), CARD 4, COLS. 1-5.

THE NUMBER OF CARDS EQUAL TO COLS. 16-20, CARD 6.

****   IF ICN(1) = 1, THE BOUNDARY PLANES ARE GIVEN IN TERMS OF

THE MILLER INDICES OF CRYSTALLOGRAPHIC PLANES AND THE PER-

PENDICULAR DISTANCES IN CM OF THESE PLANES FROM AN ARBI-

TRARY ORIGIN WITHIN THE CRYSTAL.

COLS.   1-10     H

11-30     K

21-30     L

31-40     DISTANCE IN CM.

(IF A COMPARISON IS TO BE MADE WITH ABSORPTION CORRECTIONS

CALCULATED BY GONO9, ONE MUST NOTE THAT THE CORRESPONDING

PLANES ENTERED IN GONO9 HAVE COEFFICIENTS H/D*, K/D*,

L/D*.)

****   IF ICN(1) = 2.  THE UNIQUE OBSERVED SETTING ANGLES PHI,

OMEGA, CHI NECESSARY TO BRING THE BOUNDING PLANE NORMAL

INTO THE DIFFRACTING CONDITION (ON THE XY PLANE, ALONG +Y)

ARE USED, WITH D IN CM.

COLS.   1-10     PHI (OBS)

11-20     OMEGA (OBS)

21-30     CHI (OBS)

31-40     D (CM)

ANGLE ZERO CORRECTIONS ARE MADE INTERNALLY.

DATAPP  7

****   IF ICN(1) = 3.  THE COEFFICIENTS A, B, C, OF THE EQUATION

OF PLANE:

A*X + B*Y + C*Z - D = 0

ARE USED, WITH D IN MM.

COLS.   1-10     A

11-20     B

21-30     C

31-40     D (MM)

*****NOTE:  FOR ICN(1) = 3, D MUST BE IN MM.*****

8.  OPTIONAL, DATAPP ONLY.  DATAPT AUTOMATICALLY PRODUCES THIS OUTPUT.

NEEDED IF ICN(4) = 1.

PROGRAM CALCULATES DISTANCE BETWEEN THEORETICAL

INTERSECTION OF VERTICES IVERT1 AND IVERT2.

THE VERTEX NUMBERS MUST BE KNOWN.  SINCE THE PROGRAM PRINTS

THE VERTEX NUMBERS AUTOMATICALLY,  THIS OPTION CAN BE RUN

ONLY ON THE SECOND OR LATER PASS.  IT IS INTENDED AS A

CHECK.  IF IVERT1=IVERT2, TERMINATE.

9.1 (I5,7F10.5)  REFLECTION BATCH CARDS.

COLS.    1-5     IQ  SCALE FACTOR NUMBER.  IQ WILL BE

PUNCHED ON OUTPUT CARDS IF NSCALE IS

SET TO 1 IN *IMP*; FOR OTHER VALUES

OF NSCALE THE SCALE FACTOR NUMBER IS

TAKEN EQUAL TO NSCALE.

6-15         NO LONGER USED.

16-25     C(1):  THE PROGRAM WILL CALCULATE

:  SIGMA (I-B)**2 (OBSERVED

:  VARIANCE), AS:

:  (I + B) + C(1)**2*(I - B)**2 +

:    C(2)**2*(I(CORRECTED) - I)**2.

26-35     C(2):  THE THREE TERMS REPRESENT THE

:  STATISTICAL ERROR, A PROPORTIONAL

:  ERROR AND THE UNCERTAINTY IN THE

:  ABSORPTION CORRECTION.

36-45     MU  ABSORPTION COEFFICIENT IN RECIPROCAL

CM.

46-55         SCALE FACTOR BY WHICH F**2 IS MULTI-

PLIED.  MUST NOT BE ZERO.  1.0 IS

OFTEN APPROPRIATE.

56-65     OMEGAZERO CORRECTION WHICH IS SUBTRACTED

DATAPP  8

FROM OMEGA ON CARD TO GET TRUE

OMEGA.

66-75     PHIZERO CORRECTION WHICH IS SUBTRACTED

FROM PHI ON CARD TO GET TRUE PHI.

76-80     TWOTHETA MONOCHROMATOR

9.2  (F10.5,2I10)   BATCH CARD TWO

COLS    1-10     STARTING VALUE OF THE SCALING FACTOR

USUALLY 1.0 BUT IF RUN IS BEING PICKED

UP FROM A PREVIOUS PARTIAL RUN THEN USE

THE LAST VALUE PRINTED

20 = 0 LINEAR TWO POINT INTERPOLATION FOR SCALE

= 1 FOUR POINT INTERPOLATION FOR SCALE

CUBIC POLYNOMIAL

30 =   THE NO. OF STANDARD REFLECTIONS USED

9.3 (NSTD(5X3I5)) BATCH CARD 3  INDICES OF STANDARD REFLECTIONS

COL 10     H FOR FIRST STANDARD

15     K ''   ''      ''

20     L ''   ''      ''

30     H FOR SECOND STANDARD

35     K ''    ''     ''

40     L ''    ''      ''

50     H FOR THIRD ETC.

10.  OPTIONAL.  REFLECTION DATA IF READ FROM CARDS.

INPUT IS DEFINED IN ROUTINE *IMP*.  TAPE 3 IS INPUT, OR

TAPE 5 FOR CARDS.  USER MUST SUPPLY HIS OWN *IMP*.

SEE REPLACEABLE ROUTINES FOR DISCUSSION.

OUTPUT:

BINARY TAPE 4; OR CARDS AND/OR BCD TAPE 2 WITH:

FORMAT (I1,F3.0,2F4.0,2F9.3,F3.0,6F6.3,F6.4)

LOGICAL RECORDS ARE COMPOSED OF:

JDENT = 0 NORMALLY.

= 1 ON DUMMY RECORD FOLLOWING LAST REFLECTION

H   (                )

K   ( FLOATING POINT )

L   (                )

(F**2) = (OBSI - (SLO/SLB)*BACFK)*LP*(1/ABS)*(SCALE

FACTOR SPECIFIED ON BATCH CARD)*(SCALE

FACTOR FROM SUBROUTINE SCALE).

NOTE THAT F**2 MAY BE NEGATIVE.

SIGMA(F**2) = SCALE FACTORS*LP*(1/ABS)*POISSON COUNTING

STATISTICS OBTAINED AS FOLLOWS:

SIGMA(POISSON)**2 = OBSFI + (SLO/SLB)**2*

BACFK.

ADDITIONAL TERMS CAN BE ADDED WITH INFOR-

MATION ON CARD 9.

SCALEN   SCALE FACTOR NUMBER FROM INPUT.

= IQ  COLS. 1-5, CARD 9; IF AND ONLY IF

NSCALE = 1 IN *IMP*

= NSCALE  IF

NSCALE NOT = 1 IN *IMP*

XN(1):   COMPONENTS, IN ANGSTROMS, OF THE UNIT

XN(2):   VECTOR NORMAL TO THE PLANE CONTAINING THE

XN(3):   INCIDENT AND DIFFRACTED BEAMS, ALONG

A, B, AND C.

XNSO(1):   COMPONENTS OF UNIT VECTOR NORMAL TO INCI-

XNSO(2):   DENT BEAM IN THE PLANE OF THE INCIDENT AND

DATAPP  9

XNSO(3):   DIFFRACTED BEAMS.

TBAR   MEAN PATH LENGTH FOR THIS REFLECTION--

WEIGHTED BY ABSORPTION.

(THE LAST SIX QUANTITIES MAY BE USED IN

CALCULATION OF AN ANISOTROPIC EXTINCTION

CORRECTION (COPPENS AND HAMILTON, (1968).)

REPLACEABLE SUBROUTINES:     (NOTE NAME CHANGES FOR RANDOM FILE SYSTEM)

SUBROUTINE IMPD(FH,FK,FL,S,B,SCB,OM,PHI,SLO,SLB,

OBSFI,BACFK,IDENT,NSCALE,ICN,NBACK)

DIMENSION ICN(16)

A CALL IS MADE TO THIS ROUTINE TO READ EACH REFLEC-

TION IN TURN.

FH   :

:

FK   :    MILLER INDICES, REAL

:

FL   :

S         PEAK INTENSITY

B         TOTAL INTENSITY OF ALL BACKGROUNDS

SCB       SCALE FACTOR: USUALLY = 1.

OM        OBSERVED OMEGA

PHI       OBSERVED PHI

SLO       PEAK SCAN LENGTH TIME (SEC)

SLB       TOTAL BACKGROUND COUNTING TIME (SEC)

OBSFI     IDENTICAL TO S

BACFK     IDENTICAL TO B

IDENT     DATA TERMINATION SIGNAL

NSCALE    A SCALE FACTOR NUMBER, TRANSFERRED

TO OUTPUT

ICN       ICN(16) ARE THE CONTROL INTEGERS

NBACK     NUMBER OF BACKGROUNDS OBSERVED

(NUMBER OF TERMS CONTRIBUTING TO B)

ICN(16) ARE THE CONTROL INTEGERS WHICH MAY BE USED

BY THE PROGRAMMER.  ONLY THE FIRST TEN HAVE

SPECIAL MEANING IN THE PROGRAM, THE OTHERS PROVIDE

SOME FLEXIBILITY IN DATA MANIPULATION.

DATAPP 10

FOR IDENT:

IF(IDENT .EQ. 1)  3 NEW BATCH CARDS PLUS NEW

CALLS TO IMPD WILL BE

MADE.

IF(IDENT .EQ. 2)  RUN IS TERMINATED.

IF(IDENT .EQ. 3)  A COMPLETE NEW RUN WITH

NEW TITLE, ETC., IS

STARTED.

IF(IDENT .GT. 3)  NEW CARD 10 PLUS NEW

CALLS TO IMPD WILL BE

MADE.

SET FH = FH + 100000, IF THIS IS A STANDARD REFLEC-

TION.

SUBROUTINE SCALED(IH,IK,IL,ISIG,M,NSCALE,NREJ,

JM,H1,K1,L1,H2,K2,L2,SCLA,NLINES)

THIS ROUTINE IS CALLED SHORTLY AFTER IMPD FOR EVERY

REFLECTION.

IH,IK,IL ARE INTEGER MILLER INDICES OF REFLEC-

TION.

ISIG IS THE NET PEAK COUNT, M IS THE INTE-

GRATED BACKGROUND COUNT.

NSCALE IS THE SCALE FACTOR NUMBER AS SUP-

PLIED BY IMP.

NREJ MAY BE RETURNED NOT EQUAL TO 0, AND

REFLECTION WILL BE REJECTED.

JM IS SERIAL COUNT NUMBER OF REFLECTION,

THIS CAN BE USED TO EXCLUDE INTENSITY

STANDARD REFLECTIONS FROM FINAL LIST.

SCLA IS A SCALE FACTOR--INITIALLY 1.0--

WHICH MAY BE CALCULATED BY THIS ROUT-

INE AND MULTIPLIES THE SCALE FACTOR

ON THE INPUT CARDS AND THAT SPECIFIED

ON THE BATCH CARD.

THE ROUTINE SUPPLIED DOES THE FOLLOWING:

1)  SETS NSCALE = 1.

2)  IF THE THREE STANDARD REFLECTIONS HAVE

OCCURRED TOGETHER IN ORDER AFTER THE FIRST

SUCH OCCURENCE, THE OLD SCALE FACTOR IS

DATAPP 11

MULTIPLIED BY:

OLD TOTAL NET PEAK COUNT FOR THE TWO REFLECTIONS

------------------------------------------------

NEW TOTAL NET PEAK COUNT FOR THE TWO PEFLECTIONS

INITIAL SCALE FACTOR IS 1.0.

3)  IF THIS FACTOR LIES BETWEEN 0.98 AND 1.02,

THE ROUTINE RETURNS WITH THE OLD SCALE FAC-

TOR.  IF IT IS OUTSIDE THIS RANGE, THE ROU-

TINE RETURNS WITH THE NEW SCALE FACTOR.

DEFINITION OF THE WN MATRIX IN DATAPP, MULREF, AND ANDA PROGRAMS:

THE WN MATRIX RELATES THE RECIPROCAL LATTICE AXES A*, B*,

AND C* TO THE ORTHOGONAL RIGHT-HANDED GONIOMETER HEAD AXES X,

Y, AND Z.

AT ZERO VALUES OF THE SETTING ANGLES PHI, CHI, AND OMEGA,

THE AXES ARE DEFINED AS FOLLOWS:  Y LIES IN THE PLANE OF THE

INCIDENT AND DIFFRACTED BEAM AND IN THE PLANE OF THE CHI CIRCLE

IN A DIRECTION SUCH THAT THE INCIDENT BEAM ARRIVES AT THE ORI-

GIN (CRYSTAL) FROM THE POSITIVE (X), POSITIVE (Y) DIRECTION,

(X IS PARALLEL TO THE CHI AXIS, Z IS PARALLEL TO THE PHI AXIS,

SUCH THAT THE COORDINATE SYSTEM IS RIGHT-HANDED).

IF (X), (Y), AND (Z) ARE UNIT VECTORS PARALLEL TO THE

GONIOMETER HEAD AXES X, Y, Z, THEN WN IS DEFINED BY:

( (X),(Y),(Z) ) WN = @ (A)*/ABS(A*),(B)*/ABS(B*),(C)*/ABS(C*) ?

OR:

AG(TRANSPOSE).WN = ALPHA*(TRANSPOSE)

THEREFORE:

( COS(A*,X)  COS(A*,Y)  COS(A*,Z) )

WN   =   ( COS(B*,X)  COS(B*,Y)  COS(B*,Z) )

( COS(C*,X)  COS(C*,Y)  COS(C*,Z) )

DATAPP 12

Y

*                  :   INCIDENT BEAM  *

*                :                *

DIFFRACTED BEAM  *              :              *

*            :            *

*          :          *

*        :        *

*      :      *

*    :    *

*  :  *

*:*

:.  ..  ..  ..  ..  ..

X

NOTE CAREFULLY THAT THIS WN MATRIX IS RELATED TO THE

HAMILTON U MATRIX (U(H)) BY THE TRANSFORMATION:

U(H)(1,J) = A*.WN(J,1)

U(H)(2,J) = B*.WN(J,2)

U(H)(3,J) = C*.WN(J,3)

OR

U(H) = WN(TRANSPOSE).N

( A*   0   0 )

N = (  0  B*   0 )

(  0   0  C* )

END