I-1

******************************************************************************

**************  DOCUMENTATION FOR EXAFS DATA ANALYSIS PACKAGE ****************

******************************************************************************

REVISED 2-FEB-84

INTRODUCTION TO THE EXAFS DOCUMENTATION PACKAGE

The package consists of two main programs named EXAPLT and EXAFIT.  In

addition, utility programs PHASE CONHLD, MCMAST, and CURSRT are included.

These are described briefly here, and in full detail in the following

sections.

EXAPLT	This is the main analysis program that carries out all

analysis steps from raw data through Fourier transforms

in preparation for non-linear curve fitting.  It includes

several special features for analysis of the phase of

the EXAFS oscillations.  The program is highly interactive

and can write disc files of data an various stages of

analysis.

EXAFIT	This program carries out the non-linear curve fitting

to a model EXAFS equation.  Files written by the EXAPLT

program are taken as input to EXAFIT.

PHASE	This is a utility program to set up and maintain disc

files that contain tables of reference phase functions

that are obtained either from theoretical calculations

or from empirical measurements on reference materials.

The disc tables are used by both EXAPLT and EXAFIT.

CONHLD  This is a utility program to set up on the system device

the file CONST.HLD for use by EXAPLT.  Normally this program

will be executed only once in setting up the EXAFS package;

CONST.HLD is maintained by EXAPLT.

MCMAST	This is a utility program to set up on the system device

the file MCMSTR.TBL for use by EXAPLT.  Normally this program

will be executed only once in setting up the EXAFS package;

MCMSTR.TBL is maintained by EXAPLT.

I-2

CURSRT	This is a utility program for testing graphic input and output

operations on the graphic display device.  Graphic terminals

that can run this program can be expected to work with the

EXAFS package programs.

EP-1

***************************************************************************

********** DOCUMENTATION FOR THE EXAPLT, EXAFS ANALYSIS PROGRAM ***********

***************************************************************************

REVISED  4-FEB-83

INTRODUCTION: The EXAPLT PROGRAM is a FORTRAN IV program designed to read

in raw EXAFS (Extended X-ray Absorption Fine Structure) data in

the OLD or NEW KIRBY SSRL FORMAT and provide a set of analysis

options (SMOOTHLINE, FOURIER TRANSFORM FILTERING, PHASE ANALY-

SIS, AND PHASE REMOVAL) which can be performed on the data via

graphical interaction.  The resulting output from each analysis

option can be saved on disk in a special format for later recall

or be used as input for the next option chosen. The EXAPLT program

provides flexible graphics so the user can conveniently interact

with the data and control the the graphical presentation of the

results.

STRUCTURE:    The EXAPLT PROGRAM is modular in nature, and consists of options

that can be thought of as operators that act on a set of data

stored in an input space and deliver the results of the operation

to an output space.  The input space consists of a pair of storage

arrays X and Y, and the output space a pair V and W.  After any

operation the contents of V and W are plotted on the screen, but

X and Y are left unchanged.  This means that the program user,

after viewing the results of an operation, has the choice of

stepping back to the results of the previous operation, either

to try the operation again with different parameters, to correct

errors, or to try another option.  This feature gives the EXAPLT

program a powerful flexibility in that the outcome of trying any

option can help the user decide how to proceed.  When the outcome

EP-2

of an option is satisfactory, the user can elect to retain the

results with the KE option.  This loads the contents of the U,V

arrays (output space) into the X,Y arrays (input space) in pre-

paration for further processing.  Not all options produce changed

output data; options such as XP, CS, and DF that effect only the

display of the data in V,W do not alter either X,Y or V,W.  If an

option that changes V,W has been executed but is not retained by

selecting KE then PL will restore the arrays to the condition

prior to that latest operation and replot the data arrays. Other-

wise PL just replots the results of the current step.

OPERATION:    The EXAPLT program should be run from the system device in order

to not interfere with I/O on the default device.  Four files, in

addition to data files, (CONST.HLD, MCMSTR.TBL, TEOLEE.PHT, and

PARAM.PHF) are required for access by EXAPLT during execution of

several of the options.  The CONST.HLD file contains current

status information about the data being analyzed and, therefore,

should be maintained on the system device.  The file, MCMSTR.TBL,

contains tables of McMaster coefficients of X-ray cross sections

useful for for computing an energy dependent absorption normaliza-

tion which is an option in the smoothline treatment.  In the de-

fault treatment, the EXAFS oscillations are normalized to an ener-

gy independent absorption step.  The MCMSTR.TBL file should be

maintained on the system device, also.  The other two files are

phase tables, required as input during phase options, and these

can be maintained on the default device or an interchangable de-

vice such as floppy disk.  When a prompt symbol >[BELL] occurs on

the screen or teletype the program is waiting for an instruction.

A list of valid instuctions (two-letter keywords) is given below,

with short descriptions for each.  A more detailed description of

each instruction is given further on.

EP-3

HARDWARE:     Hardware requirements for the EXAPLT program call for at least a

16 bit PDP,LSI 11 processor with extended memory capability for

addressing 128K words.  A Tektronix 4010,14,25 or equivalent

graphics terminal, supporting the Tektronix Terminal Control

System (TCS) with graphical curser input, is necessary, preferably

including a screen hardcopy unit.   A hard disk or dual density

floppy drive is a prerequisite to installation. A line printer

such as VERSATEC 1200, which has optional screen copy hardware,

provides a versatile print/plot mode ideal for the fast output of

the EXAPLT program.  A floating point unit in the processor is

essential.

INSTRUCTIONS: Two-letter EXAPLT options accepted in response to >[BELL] prompt

EX --- Exit from EXAPLT program

RE *-- Read in raw EXAFS data file, two formats

OP *-- Options of channels to ratio and logarithm of ratio

PL --- Plot of current output array with current captions

EN *-- Converts stepnumber to energy, input beamline specs

SL **- Remove smoothline background, cubic spline, and

optional McMaster treatment

KV --- Converts energy to wave vector representation

FT **- Computes forward FFT,Magnitude of transform plotted

BT -*- Computes inverse FFT of a chosen window of data

KE --- Load current output array and captions into input

array. Use after EN,ES,SL,KV,DG,FT,BT,PH before

continuing to next analysis option.

XP -*- Expand a chosen section of a plot using interactive

curser. Stores the new plot window in replot stack

(up to 3 sets maintained in stack).

EP-4

RP **- Replots any of the three current plot windows de-

termined in XP or IS. Cleared when PL or rescale of

the data occurs.

IS *-- Terminal input of up to three plot window sets into

replot stack.

ES **- Terminal or graphic edge shift calibration

DG -*- Deglitching of data by pairs

PT -*- Labels point coordinates via interactive curser.

WF --- Writes current output arrays into a file on the de-

fault device with current headings, captions and

analysis map.  Special format filename depends on

current status of output data ( xxxxxx.*xx ) where

* = R   Raw data ratio file

* = L   Log of ratio file

* = A   Absorption vs Energy

* = X   (XANES) Absorption vs Energy

* = E   Chi vs Energy

* = K   Chi vs Wave Vector k

* = F   Magnitude vs Radius

* = B   Backtransform k**nChi vs k

PR --- Writes above file to line printer instead of to

default storage device.

RF *-- Reads in a special format WF file from default

device and updates all arrays, captions, and

parameters.

CS --- Produces hardcopy image of graphic screen.

PM --- Writes current file header map, which logs the

analysis parameters and constants, to the printer.

LM --- Does the same as PM except writes to the active

consol screen.

EP-5

DF --- Computes differential of input array and plots it.

Ideal for locating the edge for precise absorption

edge calibration when using ES.

PF **- Initiates phase shift comparison and fitting al-

gorithms.  Can determine R and E0 for a given phase

function.  The experimental phase can be compared

to a phase stored in the phase tables.

Details of other options available in PF decribed

later

PH *-- Prepares an array for subsequent phase removal step

prior to Fourier transforming. Uses phase tables.

The phase removal then occurs invisibly in the FT

routine, however, the imaginary part of the trans-

form is plotted with the magnitude for this option.

AF *-- Generates the and plotting of backtransform

envelopes and the calculation and plotting of the

log of the ratio of amplitudes vs k**2 after a

successful BT option.  The reference amplitude for

this algorithm may be input from terminal, tables,

or interpolated from disk data file.  Least squares

fitting to the Ln ratio curve, provides an estimate

of the relative disorder.

FC **- An option used on fluorescence spectra from concen-

trated samples.  An absorption correction factor

may be applied to the absorption spectrum based on

the value of the stepheight determined in the SL

option.

EP-6

ED *-- An option to display several near edge spectra and

difference them if desired.  Up to three edge

may be read in from disk files to be used in

differencing or display.

TI *-- An option to interpolate the amplitude or phase

from the Inverse Fourier Transform and enter the

interpolated values into the TEOLEE.TBL tables.

The range of interpolation must be coincident with

the full TEOLEE range (3.7795-15.1178).

*--    Alpha-numeric input required

-*-    Graphic curser input required

**-    Both required

EP-7

DESCRIPTIONS: Detailed descriptions and examples of each two-letter option

>[BELL]

EX<ret>

The EX option terminates the execution of the EXAPLT pro-

gram and returns to system monitor.

>[BELL]

RE<ret>

The RE option reads all columns of data present in the

specified raw data file.  Reads both 'old' and 'new' Kirby

SSRL formats.  Automatically enters the OP option. An error

message is displayed if the specified file name is not found

on the default device.

ERROR -- FILE NOT FOUND

RT-11 file names must be in the format  (xxxxxx.xxx)

EP-8

>[BELL]

OP<ret>

The OP option allows user to choose the channels to ratio

and whether the natural logarithm of the ratio is desired.

Sample dialog:

*-      OPTIONS FOR THE OUTPUT DATA ARE AS FOLLOWS:

*-             <CR> = RETURN TO MAIN PROGRAM

*-               1  = INPUT CHANNEL #1

*                . . .

*                6  = INPUT CHANNEL #6 [unity]

-               -1  = INVERSE OF CHANNEL #1

*       RATIO OF CHANNELS I(KOPT)/I(LOPT)

*       ENTER (KOPT/LOPT) OPTIONS: 1/2<ret>

(desired channels 1 and 2 are ratioed)

-       ENTER (1 OR -1) OPTION: 1<ret>

(channel 1 which is already ratioed in 'old' format)

*-      DO YOU WANT TO TAKE THE LOG? (Y/N) :  Y<ret>

- Old format dialog

* New format dialog

The selected option is automatically plotted.

>[BELL]

PL<ret>

The PL option copies the X,Y data arrays to V,W and

plots V,W  on calculated scaling. Current captions and

labelling is controlled automatically.

EP-9

>[BELL]

EN<ret>

The EN option converts stepnumber to an energy scale which

is 0. at the X-ray energy of the absorption edge. Sample

dialog:

PRESENT MONOCHROMETER STEPS/DEG. :   2000.0<ret>

PRESENT CRYSTAL LATTICE SPACING : 1.9200<ret>

STEP NUMBER OFFSET : 0.0<ret>

EDGE ENERGY (EV'S) : 8978.2<ret>

ANY CHANGES ? (Y/N) : Y<ret>

(inputs are expected as real numbers with decimal points;

default is just <ret>)

PRESENT MONOCHROMETER STEPS/DEG. : <ret>

PRESENT CRYSTAL LATTICE SPACING : <ret>

STEP NUMBER OFFSET : <ret>

EDGE ENERGY (EV'S) : 8978.8<ret>

ANY CHANGES ? (Y/N) : N<ret>

(this example changed the value of the edge energy only)

The data is rescaled and plotted on the energy scale de-

termined by the beam line parameters above.

EP-10

>[BELL]

SL<ret>

The SL option calculates a smoothline background to be

subtracted from the absorption data. The result is rescaled

and plotted.  The smoothline divides the data between chosen

endpoints into multiple regions (3 is default) and a poly-

nomial spline is fit into each region with continuity of

derivatives at the endpoints of the regions. 9 is the max-

imum allowable number regions the user can chose.  Responses

to the first graphical curser prompt are:

(1)  N<ret>   end points chosen are NOT stored for

later use.  The normal default of

reponding with ANY CHARACTER will

store the endpoint parameters and the

step height values for later use

with the response 'P'(previous).

(2)  R<ret>   The number of smoothline regions can

be input if the default of 3 regions

is not desired.  The graphic curser is

returned for input of a number between

2 and 9 inclusive then <ret>.  The

curser is returned again for end point

selection.

EP-11

(3)  P<ret>   This response is appropriate only for

the first time the graphic curser ap-

pears. It obtains previously stored

end point values and the step minimum

from disk file automatically and pro-

cedes directly through the computation

and plotting of the smoothline back-

ground removal.

(4)  L<ret>   This response requires positioning of

of the graphic curser to locate the

first end point. The response 'L' as-

sumes the last data point as the other

end point and begins immediate compu-

tation of the smoothline.

(5)  (any char.)<ret>  position curser and respond

with any character except N,R,P,L and

a cross appears at the chosen point.

The curser is then returned for the

selection of the other end point.

SL parameters and endpoints are stored

on disk file for later recall.

EP-12

Except for the 'P' response, the curser will be returned for

selection of the pre-edge minimum in the data for extrapo-

lation of the step height.  At this point the user may re-

spond to the graphic curser with any character, however, an

optional MCMASTER TREATMENT of the step height normalization

may be obtained by responding with 'M'.  The MCMASTER coef-

ficients will then be obtained from the MCMSTR.TBL file af-

ter the next curser is responded to.  For all responses, the

step height is printed on the plot.  After the step height

selection, another graphic curser will appear allowing the

user to make a hardcopy of the screen containing all of the

smoothline information.   A response of 'E' to this final

curser will terminate the graphic smoothline steps. A

response of 'X' will also terminate the graphic steps but

output of the smoothline option will be a normalized

XANES plot rather than a CHI vs ENERGY plot. Any other

response to this final curser will cause the sequence of

steps for the selection of STEP MINIMUM, MCMASTER, and XANES

to be repeated.

(6)  M<ret>  is the reponse to the step height cur-

ser after locating the pre-edge min-

imum to initiate a MCMASTER TREATMENT

of the step height determination.

After response to the final curser

the screen will clear and a dialog

prompting for the Mcmaster coef-

ficients will appear (see sample

dailog below).

EP-13

(7)  E<ret>  is the response to the graphic curser

which is returned after the step

height calculation. This response

ends the smoothline step unless the

MCMASTER TREATMENT is specified by

'M' response to the previous curser.

In which case, this 'E' response

begins the dialog for obtaining the

MCMASTER coefficients.

(8) X<ret>   is the response to the final SL curser

which will envoke the display of the

normalized edge spectra or XANES

rather than the display of the CHI

vs ENERGY plot.  The XANES may be

further treated as an ABSORPTION

vs ENERGY data file but with a

xxxxxx.Xxx extension name instead

of a xxxxxx.Axx extension name.

Any other response returns the cursers

again for a repeat of the step height

determination sequence.

EP-14

(1) and (3) must be used prior all other responses if the

number of smoothline regions desired is other than 3.

A sample dialog for the MCMASTER TREATMENT is given here:

FOR MCMASTER TREATMENT, YOU MUST SPECIFY THE MCMASTER

COEFFICIENTS OF THE ABSORBER EDGE AND THE BACKGROUND EDGE.

TYPE ATOMIC NO. OF ABSORBING ELEMENT AND EDGE NO.

(11-103),(K=0,L=1,M=2,N=3) AND RETURN

NATOM,NEDGE ? 78,1<ret>

THE MCMASTER COEFFICIENTS FOR THE

PT L EDGE ARE: 17.3636   -2.2111     -.0731     0.0000

OK? Y<ret>

TYPE ATOMIC NO. OF BACKGROUND ELEMENT AND EDGE NO.

(11-103),(K=0,L=1,M=2,N=3) AND RETURN

NATOM,NEDGE ? 78,2<ret>

THE MCMASTER COEFFICIENTS FOR THE

ARE:  0.0000    0.0000     0.0000     0.0000

OK? N<ret>

MCMASTER TABLE ENTRY NOT SPECIFIED OR REQUIRES CHANGE

ENTER NAME SPECIFYING THE X-RAY ABSORPTION EDGE :

PT M EDGE<ret>

TYPE COEFFICIENT CHANGES WITH SEPARATING COMMAS

16.7024,-2.71631,0.,0.

THE MCMASTER COEFFICIENTS FOR THE

PT M EDGE ARE: 16.7024   -2.7163     0.0000     0.0000

OK? Y<ret>

In this case the McMaster coefficients were obtained for

a normalization of a Pt L edge.  The Pt L edge Mcmaster

coefficients were located in the MCMSTR TABLE on disk and

displayed for verification.  The coefficients for the back-

ground Pt M edge were not previously entered into the table

so the user must input the entry name and table values.

The typed coefficients are again displayed for verification

prior to the computation of the step height normalization.

A reponse of 'N' allows the user to modify the coefficients

for that table entry.

EP-15

>[BELL]

KV<ret>

The KV option makes the conversion of energy to photo-

electron wave vector representation (Reciprocal Angstroms).

The result is plotted with current headings and captions.

>[BELL]

FT<ret>

The FT option computes a forward FFT of a desired re-

gion of data in k-space.  The routine also interpolates

the data on an even grid of 96./(N) where N is chosen by

the user to be 2048, 4096, or 8192 prior to the transform.

If the phase removal option is in effect, a set of ins-

structions, invisible to the user, removes the effect of

a complicated phase shift. The transform magnitude as a

function of the radial harmonics is plotted.  Also, when the

phase removal option is in effect, the imaginary part of the

transformed data is plotted along with the magnitude. A

choice of windowing factor (% Hanning Window) and weighting

factor k**n are input by the user.

EP-16

Sample dialog :

(graphic curser is presented for selection of transform

range in k-space, acceptable responses are:)

N<ret>            Endpoints and input parameters will not be

stored for later use. The default is save.

Curser returns for selection of other end-

point.

P<ret>            Previously saved endpoints and parameters

are used for the transform.

(any char)<ret>   Locate curser and respond with any charac-

ter except N and P to choose endpoints.

(After endpoint selection the terminal prompts for % window-

ing, k weighting and the number of points to be transformed)

%  10<ret>

K**  1<ret>

#  4096<ret>      NOTE: just a <ret> at this prompt causes

the default of 4096 to be chosen.

a <1> at this point let you repeat

the dialog and reselect the

endpoints.

(This example sets up a forward transform of 4096 points

with the outside 10% of the data windowed by a Cosine

Hanning Window)

The resulting radial transform scaled and plotted.

EP-17

>[BELL]

BT<ret>

The BT option performs a an inverse (or back) FFT on

a determined region of the radial transform data. This

Fourier filtering step uses graphical curser input only.

No windowing is applied on the inverse transform, but a

back-transform window may be an option in the future.  When

the first curser appears the acceptable responses are 'N',

'P', and ' (any char) ' with the same meaning as in the

forward transform routine.  After selecting two endpoints

a third curser appears to hold the screen for hardcopy (if

desired).  At this time the endpoints may be reselected by

responding 'R' to the third curser.  The selection process

is repeated as above.  Any other response resumes the com-

putation of the backtransform.  The result K**N*CHI vs K is

plotted.  N is the weighting chosen in the FT option.

>[BELL]

KE<ret>

The KE option keeps the current output arrays by loading

them in to the input arrays in preparation for the next

step.  In this input-output array mode certain steps may be

executed repeatedly until a desired form of the data is ob-

tained.  The ability to back up and try again is of parti-

cular value when testing the outcome of various analysis op-

tions, such as SMOOTHLINE and FOURIER TRANSFORM steps.

NOTE: failure to keep the output array before the next step

causes the previous input array to go to the next option.

This may result in fatal errors some of the time.  In order

to continue, KE must be used after: EN,ES,SL,KV,DG,FT,BT,PH,

and FC. Auto-keep is being developed for future versions.

EP-18

>[BELL]

XP<ret>

The XP option uses the graphic curser to determine a

region of a plot to expand.  The new plot window is stored

in a replot stack having three levels.  In addition, up to

3 sets of plot windows can be stored and recalled from disk

file. Curser appears and the acceptable options are:

(any char)<ret>  Locate curser at desired lower left corner

of the new expand window. A box corner is

drawn. Curser is returned again for selec-

tion of upper corner of the expand region.

S<ret>           Locate curser at desired lower left corner

of the new expand window.  Response of 'S'

indicates that the windows values will be

stored in one of three storage locations

on disk.  The response to the second curser

determines which storage location is used.

1<ret>           Window values (1) recalled from disk when

this response is given to the first curser.

Window values (1) stored on disk when this

response is given to the second curser af-

ter a reponse of 'S' to the first curser.

2<ret>           Follows the same procedure.

3<ret>           Follows the same procedure.

Screen clears and expanded plot appears.   PL returns the

original plot.

EP-19

>[BELL]

RP<ret>

The RP option replots any of the last three expanded plots

where the original plot has not been been rescaled or PL has

not been executed.  The replot stack is reinitialized when

PL is executed.  The replot stack is particularly useful for

quickly obtaining various scaled blowups of the absorption

edge.  A prompt @ is given and the acceptable responses are:

*<ret>     most recent expanded plot

*-1<ret>   2nd most recent expanded plot

*-2<ret>   3rd most recent expanded plot

The screen is cleared and the data is replotted on the

chosen scale.

>[BELL]

IS<ret>

The IS option allows the user to enter sets of plot win-

dows from the terminal keyboard into the replot stack.  The

stack holds three sets of windows for replotting until the

stack is reinitialized by new scaling or execution of PL.

New entries to the stack are prompted for and the new scale

is used in the replotting.

EP-20

>[BELL]

ES<ret>

The ES option produces a shift in the V output array of

the data so that the horizontal scale is zero at the point

specified by the graphic curser.  This is a particularly

useful option for energy edge calibration.  When the curser

appears the acceptable responses are:

(locate curser and type:)

N<ret>           The edge shift is not stored for later use.

P<ret>           Recalls previous edge shift

I<ret>           Terminal input of edge shift value

L<ret>           Terminal input of edge shift value loaded

into storage location on disk, no edge

shift is executed on the data.

(any char)<ret>  Normal execution of edge shift, stores

edge shift in disk file.

ES always produces a new plot on the shifted scale but

will only be retained after using the KE option.

>[BELL]

DG<ret>

The DG option uses the graphic curser for interactive

deglitching of the data.  DG allows easy multiple deglit-

ching.  The deglitching is terminated by the 'E' response

after an integral number of pairs of cut points are de-

termined using the graphic curser.  The deglitched regions

are replaced by a linear interpolation between pairs of cut

points and the deglitched data is replotted for inspection.

The KE option retains the deglitched data.

EP-21

>[BELL]

PT<ret>

The PT option uses the graphic curser to label the (X,Y)

coordinates of points on the plot.  The printed coordinates

usually avoid being written on the data.  Locate the curser

and return any character key in response.

>[BELL]

WF<ret>

The WF option writes a special format file to the de-

fault device for later recall using the RF option.  The file

contains the current input array and header to update the

parameters and captions.  The filename used for the file

depends on the current status of the data.  The format is

xxxxxx.*xx where

* = R        Raw ratio file

* = L        Log ratio file

* = A        Absorption vs Energy

* = X        (XANES) Absorption vs Energy

* = E        Chi vs Energy

* = K        Chi vs Wave Vector K

* = F        Fourier Transform Magnitude vs R

* = B        Backtransform K**n*CHI vs K

* = P        Phase function extracted in PF

The special format files can be recalled from disk in either

the EXAPLT or EXAFIT programs.

EP-22

>[BELL]

PR<ret>

The PR option writes the same special format file to the

printer instead of the default device.  The header contains

the current parameters, captions, and data.

>[BELL]

RF<ret>

The RF option reads in a special format data file and up-

dates all arrays and labels.  The file name must be one of

the following  xxxxxx.*xx , where acceptable characters in

place of * are R,L,A,X,E,K,F, and B .  An error message is

displayed if the filename is in error or the filename is

not on the default device.

ERROR -- FILE NOT FOUND

>[BELL]

CS<ret>

The CS option initiates a hard copy of the the graphic

terminal screen if a hardware screen copy unit is available.

A pause of about 20 seconds occurs before the > prompt

reappears. This pause is for compatibility with slow

hardcopy units.

EP-23

>[BELL]

PM<ret>

The PM option prints to the line printer a copy of the

the current header map only.  The header contains the

current parameters, captions, and data map.

>[BELL]

LM<ret>

The LM option lists to the terminal screen a current map

header. The header contains the current analysis parameters,

captions, and data map.

>[BELL]

DF<ret>

The DF option computes a divided difference differential

of the W output array and plots the V,dW arrays on the new

scale.  Ideal to use for locating the maximum slope assoc-

iated with the absorption edge for the calibration of the

edge. This differential cannot be kept and PL returns to the

original V,W plot.

EP-24

>[BELL]

PF<ret>

The PF option provides most of the EXAFS phase analysis

and is used after the phase of the EXAFS oscillations is

extracted in the back-transform BT.  The choices within the

phase analysis and fitting coding are numerous. The plotting

for this option is self-contained in the PF module so that

consecutive executions of the PF option is possible.  The

phase tables are accesssed frequently during the execution

of PF coding and should be maintained on the default device.

The choices within PF include display of phase data and

functions, list of phase data and fitting of phase data.

The user may choose to display the experimental phase,

display a phase stored in the tables TEOLEE.PHT, PARAM.PHF,

or display both at the same time.  The PHASE PROGRAM is

used to construct the phase tables and the entries therein.

It is also possible to store the phase in a normal

EXAFS file.

The user may choose to perform three types of fits to

the phase data.  One is a linear least-squares fit of a

parameterized phase function

phi(k) = 2kR + A0 + A1k' + A2k'**2 + A3/k'**3

to the experimental data, where the An's are determined by

by the best least-squares fit, provided that the values of

R and E0 (k'=sqrt[k**2 - .262467]) are known by the user.

The coefficients An and E0 can be displayed on the screen

and if desired can be entered into the phase tables for

later reference.  The second type of fit is a non-linear

EP-25

least-squares iterative fit of a known phase function

(only parameterized table or terminal input) to the data in

order to determine R and E0.  Of course, a change in R is

equivalent to a change in the slope of phi(k) which has a

large effect on the phase function a larger values of k. The

change in E0 is proportional to 1/k in the phase and thus

has its greatest effect at smaller values of phi(k).  In

this respect the two parameters are at most weakly coupled,

and therefore ought to converge to a reasonable fit within

a few iterations.  The last type of fitting in the phase

module is linear least squares fitting to a phase residual

(difference between an experimental phase and a known phase

from the tables or input from the terminal) for modeling

disorder effects on the EXAFS phase.  The phase residual is

modeled by linear and cubic terms (Eisenberger et.al.)

delphi(k) =  C1k + C3k**3

where the coefficients  C1 and C3 model the effects of dis-

order on the EXAFS phase.  C1 is related to the thermal ex-

pansion of the nearest-neighbor bonds. This fitting method

is informative provided that reference phase shifts are well

characterized and fit the data well for small disorder.

EP-26

Sample PF dialog:

(D)ISPLAY ONLY OR DISPLAY AND (F)IT ? D<ret>

(L(ist) will list the phase data on the printer)

(E)XPERIMENTAL OR (T)HEORETICAL PHASE OR (B)OTH ? B<ret>

(S at this point will interpolate and store the phase at

N points, specified in a specific dialog)

DO YOU WANT TERMINAL (I)NPUT OF YOUR KNOWN PHASE OR A

(P)ARAMETERIZED OR (T)ABULAR PHASE FROM DISK TABLES ? T<ret>

(S at this point gives the possibility to shift the phase

at 2 PI)

PLACE TABLES DISK IN DISK DRIVE AND THEN

SPECIFY ABSORBING ELEMENT, QUANTUM NO. L : PT,2<ret>

SPECIFY SCATTERING ELEMENT : PT<ret>

ABSORBER:

PT2      xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

SCATTERER:

PT       xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

OK? Y<ret>

TYPE IN E0 FOR YOUR TABLE: 0.0<ret>

TYPE IN R1 FOR YOUR TABLE: 2.774<ret>

This example shows the dialog for displaying both Pt

experimental and reference tabular phase shifts on

the same plot.  The phase tables are accessed for the

entries PT2 and PT to construct the reference phase

shift and is shifted appropriately by E0 if specified

by other than zero. The R1 value is specified for the

subtraction of 2kR1 from the experimental EXAFS phase.

Error messages occur if the phase tables are not found

on the default disk or if a particular entry is not found

in the tables.  The PF option will then terminate and the

> prompt will reappear.  A successful table search will

result in the display of the table entry values for

verification.  Table entries can only be modified in the

PHASE PROGRAM, so the user must exit EXAPLT if changes are

desired.  We suggest that the reference phase shifts be

loaded into the tables and carefully verified before the

PF option is used in EXAPLT.

EP-27

>[BELL]

PH<ret>

The PH option sets up the phase removal of a known phase

shift from the data prior to Fourier transforming.  Access

to the phase tables is required.  The user is prompted to

choose the input phase shift to be entered from tables or

manually from the terminal keyboard.  The phase shift to be

removed is stored in an array which is passed invisibly

to the Fourier transform.  The PH option should be used

after the conversion to the wave vector representation and

before the Fourier transform.  The dialog is similar to the

phase table dialog in the PF option, however, the user must

specify whether to remove or add effects of scattering phase

shifts.  The Chi vs K oscillations are then replotted

in preparation for the Fourier transform, however, it is

necessary to use the KE option to keep the phase shift array

prior to using PF.  If the phase shift was determined from

tabular entries then the range of data available for trans-

forming is limited to the range of the tabular phase func-

tion in k (~ 3.8 - 15.2).  The phase removal option proceeds

invisibly to the user during the Fourier transforming until

the plot is made.  In this option the imaginary part of the

Fourier transform is plotted along with the magnitude of the

Fourier transform.

EP-28

>[BELL]

AF<ret>

The AF option provides a comparison of experimental and

reference/empirical amplitudes by simple display of the

envelopes or by the LN RATIO of the amplitudes technique.

A reference amplitude may be input from the terminal in

functional form  AMPL(K)=N/KR**2*F(K)*exp(-2K**2R**2)  with

F(K)= A/(1+B**2(K-C)**2), tabular form from disk tables, or

interpolated from an input   K**N*CHI file (xxxxxx.Bxx) on

disk.  The internal options in AF allow display of the

the amplitude envelopes or the LN RATIO of amplitudes

vs k**2 and provide least squares curve fitting to obtain

the coefficients of

LN RATIO= C0+C2K**2+C4K**4

where the intercept C0 is  LN( N/R**2 / No/Ro**2 ), the

slope C2 = -2 SI**2 (SI**2 is associated with the EXAFS

DEBYE WALLER FACTOR in the case of small disorder) and

and the quadratic term C4 is an increasing effect of large

disorder.  Coding is provided for scaling and weighting the

reference amplitude prior to displaying the envelopes or

computing the LN RATIO.  A response to the weighting dialog

of  /<ret> assumes a default of no weighting of the

reference amplitude.  For some cases, it may be useful to

fix C0 and only determine C2 and C4 from least squares fits.

Coding is provided for this.  Manual fit of the coefficients

C0, C2, and C4 is an option as well as the selection of the

fit range in K**2.

EP-29

The selection of 'Beat-adjustment' analysis in AF allows

the extraction of the backscattering amplitude of a single

shell from the backtransformed data of two close-lying

shells of the same backscatterer type.  The approximate

formula used is

AMP(1 shell)= AMP(2 shells)/CORRECTION

where

CORRECTION= SQRT{1+C**2+2COS[2(R2-R1)K]}

and

C=(R1/R2)**2 * EXP[-2(SIGM2-SIGM1)*K**2].

The necessary input for this adjustment is R1,R2 (first and

second shell bond distance) and SIGM1,SIGM2 (first and

second shell disorder factor).

EP-30

>[BELL]

FC<ret>

The FC option provides an absorption dependent correction

to a fluorescence spectrum which has been removed off of the

background absorption level using the 'X' option (XANES) in

in SL.  After FC is selected the plot clears and a dialog

appears which prompts for the fluorescence correction

factor "A".   A is determined from density and thickness

relationships in the fluorescence sample.  This correction

reduces the measured absorption in proportion to the

absolute absorption above the background.  In dilute

concentrated samples or thin samples the correction is

negligible.  If Mcmaster treatment of the XANES was chosen

in 'SL' then the same Mcmaster stepheight must be used

to reconstruct the absorption height.  The Mcmaster

coefficients obtained with the same dialog as in 'SL'.

The corrected spectrum is then replotted for

inpection or furthur processing.

EP-31

>[BELL]

ED<ret>

The ED option provides a multiple curve plotting step

for simultaneously displaying up to four edge or near edge

absorption or fluorescence spectra.  In addition, the

difference spectra may be displayed rather than the edge

spectra.  Difference spectra are simply the normalized

absorption spectra with a reference absorption spectrum

subtracted.  The energy scales may be shifted to account for

chemical differences (shifted peak positions). Normally, the

choice of energy coincidence is made based on minimizing the

the area under the difference curve in a specific energy

region.  A sample dialog is provided:

Assume for this dialog that the data file in the EXAPLT

program is a CUO (copper oxide) absorption edge. It is then

file #0 for the ED option.

(P)LOT EDGES OR (D)IFERENCE EDGES? : D<ret>

ENTER NUMBER OF INPUT FILES (0-3 ;DEFAULT=0) : 3<ret>

ENTER REFERENCE FILE NUMBER (0-3) : 3<ret>

TYPE ENERGY RANGE  EMIN,EMAX ;DEFAULT (/)=MAX RANGE: /<ret>

PLEASE WAIT A MOMENT FOR SPLINE COMPUTATION

TYPE INPUT FILE NAME #1 : CUCL.X35<ret>

TYPE EDGE SHIFT #1 : 0.<ret>

PLEASE WAIT A MOMENT FOR SPLINE COMPUTATION

TYPE INPUT FILE NAME #2 : CUCL2.X35<ret>

TYPE EDGE SHIFT #2 : -3.5<ret>

PLEASE WAIT A MOMENT FOR SPLINE COMPUTATION

TYPE INPUT FILE NAME #3 : CUFOIL.X05<ret>

TYPE EDGE SHIFT #3 : 0.<ret>

PLEASE WAIT A MOMENT FOR SPLINE COMPUTATION

This discribes an example of multiple display of 4

difference spectra.  The difference spectra will be

plotted as follows:

difference curve #0  CUO   -  CUFOIL   unshifted

difference curve #1  CUCL  -  CUFOIL   unshifted

difference curve #2  CUCL2 -  CUFOIL   shifted by -3.5eV

difference curve #3  CUFOIL-  CUFOIL   unshifted

EP-32

A similar dialog occurs for the edge plot mode, but

the prompt for reference file number does not occur.

Scaling is provided in the coding and error messages

for file specification errors.  Only those files

prior to a file specification error will be plotted

or differenced.  To re-display the edges simply

respond with  'ED' to the next > prompt after the

initial edge display.  A response of 'PL' causes

a return to the state prior to the first 'ED' response.

If difference spectra are displayed then a 'KE' response

after a successful display will retain the difference

curve #0 for further processing.

ERROR - FILE NOT FOUND OR ERROR SPECIFYING FILE

ERROR - WRONG FILE TYPE FOR EDGE OR DIFFERENCE DISPLAY

EP-33

>[BELL]

TI<ret>

The TI option contains coding for interpolating the

amplitude or phase constructed from the inverse Fourier

transform step.  The interpolation is over the TEOLEE

range and can be stored as an entry in TEOLEE.TBL tables.

The phase and amplitude data must coincide with the range

in K (3.7795-15.1178) for the interpolation to execute.

Prompts for amplitude weighting and the removal of 2KR

from the phase are similar to the 'AF' and 'PF' options.

There is a prompt for Table Entry Name (2-letter element

name) for each entry.  The coding allows a new entry to

supercede an old entry only when you specify it. Coding

for 'beat-adjustment' in two shell amplitudes is iden-

tical to the 'AF' option.

EP-34

NOTES:   This section is for comments and suggestions

BE SURE TO REMEMBER THE KE OPTION AFTER EN,DG,ES,SL,KV,FT,BT,PH,FC

THE VALUES STORED IN THE HEADER (MAP) FILE, USED FOR THE HEADER OF

THE SPECIAL FORMAT FILES WRITTEN TO DISK AND AS A RECORD OF THE

ANALYSIS, COULD POTENTIALLY DIFFER FROM THOSE ACTUALLY USED IN THE

DATA ANALYSIS IF THE 'N' (NOSAVE) RESPONSE IS USED WITHOUT DIS-

CRETION.

SPECIAL FORMAT FILES SHOULD BE WRITTEN AT KEY PLACES IN THE ANALY-

SIS SO THAT IF FATAL ERRORS OCCUR, THE ANALYSIS CAN BE RESUMED

RATHER THAN STARTED FROM SCRATCH AGAIN.  WE SUGGEST WRITING AT THE

ABSORPTION VS ENERGY STAGE AND AT THE CHI VS K STAGE.  THIS WAY

ANALYSIS OPTIONS SUCH AS SMOOTHLINE AND FOURIER TRANSFORMING MAY

MAY BE RETRIED WITH MINIMUM LOSS OF TIME AND EFFORT.

IF A FOURIER TRANSFORM IS WRITTEN TO THE DISK FILE THEN IT MAY

ONLY BE RECALLED TO THE PROGRAM FOR DISPLAY.  THE LARGE COMPLEX

FOURIER TRANSFORM ARRAY IS NOT RESTORED, ONLY THE MAGNITUDE.

ANY SUBSEQUENT BACK-TRANSFORM, REQUIRING THE USE OF THE COMPLEX

ARRAY GENERATED IN THE FORWARD TRANSFORM, CANNOT BE SUCCESSFULLY

EXECUTED.  THIS MEANS THAT PHASE AND AMPLITUDE INFORMATION IS ONLY

OBTAINED AFTER A SEQUENCE OF FORWARD AND INVERSE TRANSFORMS IS

EXECUTED.  COMPLEX PHASE AND AMPLITUDE IS NOT RESTORED BY SIMPLY

READING IN A BACKTRANSFORMED (.B*) FILE

ERRORS OCCUR WHEN 8 OR 9 REGIONS IS SPECIFIED IN 'SL' , THIS PROB-

LEM HAS NOT BEEN CORRECTED

EP-35

PT DISPLAYS THE COORDINATES OF THE CHOSEN SCREEN POINT AND IT

GENERALLY AVOIDS WRITTING ON THE GRAPHICAL DATA.  SOMETIMES IT

FAILS WHEN THE SCREEN IS VERY CROWDED AND PRODUCES UNPREDICTABLE

RESULTS AND SOMETIMES GOES INTO AN INFINITE SEARCH LOOP.  USE

WITH CARE AND ON A LESS CROWDED SCREEN.

A SPECIAL FLAG CONTROLS THE OVERPLOTTING OF THE IMAGINARY AND

MAGNITUDE OF THE FOURIER TRANSFORM IF THE PHASE REMOVAL  (PH)

IS IN EFFECT.  IF THE TRANSFORM IS RETAINED WITH (KE) THEN A

SUBSEQUENT (BT) MUST BE EXECUTED TO CLEAR THE FLAG, OTHERWISE THE

OVERPLOTTING WILL CONTINUE TO OCCUR EVEN IF NEW DATA IS READ IN

AND OTHER OPTIONS EXECUTED.  IF THE PHASE REMOVED TRANSFORM IS NOT

RETAINED WITH (KE) THEN SIMPLY TYPE (PL) TO RESTORE NORMAL

PLOTTING.

EP-36

INDEX OF EXAPLT DOCUMENTATION

Pages

Introduction                   ..............................   1

Structure                      ..............................   1,2

Operation                      ..............................   2

Hardware                       ..............................   3

Instructions                   ..............................   3,4,5,6

Description of Options

AF --- Amplitude fitting       ..............................   28,29

BT --- Back transform          ..............................   17

CS --- Copy screen             ..............................   22

DF --- Edge differential       ..............................   23

DG --- Deglitghing             ..............................   20

ED --- Edge differencing       ..............................   31,32

EN --- Energy conversion       ..............................   9

ES --- Energy shift            ..............................   20

EX --- Exit                    ..............................   7

FC --- Fluorescence correction ..............................   30

FT --- Forward transform       ..............................   15,16

IS --- Stack input             ..............................   19

KE --- Keep                    ..............................   17

KV --- Wave Vector conversion  ..............................   15

LM --- List map                ..............................   23

OP --- Option                  ..............................   8

PF --- Phase fitting           ..............................   24,25,26

PH --- Phase removal           ..............................   27

PL --- Plot                    ..............................   8

PM --- Print map               ..............................   23

PR --- Print file              ..............................   22

PT --- Point coordinates       ..............................   21

RE --- Read raw data files     ..............................   7

RF --- Read special files      ..............................   22

RP --- Replot Stack            ..............................   19

SL --- Smoothline Background   ..............................   10,11,12,13,14

TI --- Table Interpolation     ..............................   33

WF --- Write special files     ..............................   21

XP --- Expand plot             ..............................   18

Notes and comments             ..............................   34,35

Index                          ..............................   36

EF-1

***************************************************************************

******** DOCUMENTATION FOR THE EXAFIT, EXAFS CURVE FITTING PROGRAM ********

***************************************************************************

Revised  05-Nov-82

INTRODUCTION: The EXAFIT PROGRAM is a non-linear, multi-shell least squares

curve fitting program for modeling EXAFS oscillations in wave

vector (K) space.  The EXAFIT program is written entirely in

Fortran IV.  The model EXAFIT function is given by

K**M+1 X(K) = K**M SUM [ Ni/Ri**2  Ai/{1+Bi**2(K-Ci)**2}

x exp{-2Ri/(GiK+Hi)} exp{-2(KSIi)**2}

x sin(2KRi+P0+P1K+P2K**2+P3K**-3) ].

The EXAFIT program, modeled after the Lytle EXAFS fit program

(currently running in batch mode on an IBM 370), has been re-

developed and adapted to run in an interactive RT-11 environment.

This program, which stands alone except for the input data files,

occupies ~ 20K of core memory (program and overlay storage) and

more than 30K of extended memory (array storage).  The EXAFIT pro-

gram requires much less user interaction than is required in the

EXAPLT program with the bulk of the user intervention occuring

during the setup of the fits.  In setting up the fits the user is

prompted to choose which parameters to vary and is then prompted

to provide best estimate values for these parameters. The capabil-

ity to store a setup for later recall is one of the convenient

features of this RT-11 adapted curve fitting program.

The chosen parameters are refined by an iterative least-

squares procedure until a desired degree of fit is achieved or

until divergence of fit occurs.  To maintain convergence of the

varied parameters, a damping factor is applied to the system of

EF-2

independent equations, the magnitude of which is selected by the

user.  Where initial estimates may be poor, a larger damping

factor assures that the refinements don't overcorrect and diverge.

The damping factor can be relaxed as a good fit is approached.

OPERATION:    The EXAFIT PROGRAM should run from the system device so as not

not to interfere with the program I/O.  The input data files to be

read into the EXAFIT program during the setup should be maintained

on the default I/O device.  Room should be left on the default

device for several setup storage files and for scratch storage of

the coefficient matrix regenerated for each iteration.  This space

amounts to less than 100 blocks in most cases.   The only accept-

able data files to be read into the EXAFIT program are the spec-

ially formatted data files produced in the EXAPLT or TESTFN pro-

grams.  A utility program is necessary for adapting any other file

formats into the special file format.

HARDWARE:     Hardware requirements for the EXAFIT program call for at least a

16 bit PDP,LSI 11 processor with extended memory capability for

accessing 128K words.  A Tektronix 4010,14,25 or equivalent term-

inal supporting the Tektronix Terminal Control System (TCS) is

necessary, preferably including a compatible screen copy unit.

A hard disk or dual density floppy drive is a prerequisite to

installation.  A line printer such a VERSATEC 1200, which has

optional screen copy hardware, provides a versatile print/plot

mode ideal for the fast output of the EXAFIT program.  Floating

point hardware in the processor is essential.

EF-3

SETUP:        The setup routine in the EXAFIT program is an important routine

for user interaction because it includes the selection of all the

fit parameters, input of the data, output of the setup map, and

the storage of setup parameters.   The setup coding is modeled

after the card image stack (1-8) used for batch mode execution of

Lytle's EXAFS fit program, however, in this program the user

selects the desired card image and then is interrogated to input

the parameters to complete the card setup.  The efficiency of the

card setup is improved by the ability of the user to recall a

previous setup and to default through the interrogation.  The card

setup coding is terminated by selecting CARD 0 and control is

passed to read in the data, to print the map, and to execute the

fits.

EXECUTION:    The input card parameters depend on the card image chosen (0-8,

0 - terminates card input).  The cards and their associated par-

ameters will be presented one-by-one below.  First, the dialog

leading to the card setup will be discussed.  Be careful that the

default device contains all of the necessary data files and stored

setup files (if any) prior to program execution.  Sample dialog:

(Run the EXAFIT PROGRAM)

.R EXAFIT<ret>             ;begins execution

DO YOU WANT TO READ CARD SETUP PARAMETERS FROM DISK?

(The response to this question determines whether a previously

stored setup is placed active.  To stop or exit the EXAFIT program

just type a   ^C at any time)

N<ret>                     ;no previous setup desired,

control passed to card selection

****

Y<ret>                     ;previous setup desired

EF-4

DO YOU WANT TO LOOK AT A TABLE OF SETUP DATA FILES?

(The response of N<ret> to this question, skips to the next

question)

Y<ret>                     ;setup file table listed on

terminal

TABLE OF CURRENT SETUP FILES ON DEFAULT DISK

SETUP FILE 1        CUFOIL.B01

SETUP FILE 2        FEFOIL.B02

SETUP FILE 3        < UNUSED >

SETUP FILE 4        < UNUSED >

SETUP FILE 5        < UNUSED >

SAVE SETUP FILE FOR ABNORMAL TERMINATIONS:

SETUP FILE 6

(In this example the setup files for fits to CUFOIL.B01 and

FEFOIL.B02 are the only ones residing on the default device

and only one of them may be chosen to be active.  If no setup

files are found on disk then an error message occurs

NO SETUP FILES ON DEFAULT DISK

and the program is restarted from the beginning)

TYPE NUMBER OF SETUP FILE FOR INPUT (1-5)

(Again, only 1 or 2 are acceptable choices other choices cause

an error message

SETUP FILE     "N"  NOT FOUND ON DISK

and prompt for another look at the setup table)

1<ret>                    ;setup file for CUFOIL chosen

****

(At this point control is passed to the card selection and setup

coding.  Card 1 is usually chosen first because certain parameters

chosen in Card 1 affect the other cards. One example is the number

shells to be fit.)

TYPE INPUT CARD NUMBER (0-8) : 1<ret>

A list of the each specific card dialog is contained in the next

section.

EF-5

CARDS:        This section contains specific card descriptions and sample

dialog.  In most cases, the card parameter values displayed can

be defaulted or changed by typing to the terminal.  For virtually

every prompt the response /<ret> constitutes a default to the

displayed parameter value.

CARD 1 dialog:

CASENAME:  CUFOIL.B01 TYPE CHANGE: /<ret>

(Casename specifies input data file to be fit)

NO. OF SHELLS:   1 TYPE CHANGE: /<ret>

(This number determines the number of shells [ the EXAFS function,

above, is summed over the number of shells with a maximum of 10 ].

Normally 1 or 2 shells is specified.)

NO. OF DAMPS PER ITERATION:  10 TYPE CHANGE: /<ret>

(This number specifies the number of times a damping factor is

applied during one iteration.  In this example the damping factor

is varied 10 times from a minimum value of 1.0 to a maximum value

(>1.0) specified by the user.  For each iteration, the results of

each damping factor application are tested for best fit and are

used to seed the next iteration.)

NO. OF ITERATIONS:  10 TYPE CHANGE: /<ret>

(This parameter has no other purpose than to limit the iteration

process.  In general, a fit converges within 10 iterations, how-

ever, it is conceivable that a complicated multi-shell fit could

take many iterations.  Our experience has been that 10 iterations

provides a good stopping point from which to evaluate whether to

continue the iteration process.)

EF-6

FIT INCREMENT:   5 TYPE CHANGE: /<ret>

(This number determines the grid on which the model EXAFS function

is calculated.  In this example every 5th data point is used in

the calculation.  Of course, the time of calculation is propor-

tional to the number of points to be calculated so it is recom-

mended that 4 or more be specified unless the data contains ra-

pidly varying oscillations.)

DAMPING FACTOR:  0.100 TYPE CHANGE: /<ret>

(The damping factor specified here is not actually the maximum

damping factor applied to the iterations but rather is the number

added to 1.0 to obtain the maximum damping factor.)

PERCENT CHANGE STOP:   0.010 TYPE CHANGE: /<ret>

(This number determines the desired goodness of fit.  When all

parameters change by less than this specified percent the itera-

ting is terminated.  A reasonable range of values is 0.010 to

0.100 percent where smaller values increases the number of

iterations required for the fit.)

MULTPLY DATA BY Q*K**N

Q:    1.000 TYPE CHANGE: /<ret>

N:    0.000 TYPE CHANGE: /<ret>

WEIGHTING*CHI=K**M*SUM[ ]

M:  -1 TYPE CHANGE: /<ret>

(These parameters determine the weighting of both the data and

the model EXAFS function.  The combinations of the weightings are

important when using standard or reference spectra for determining

the amplitude envelope in a given shell.  For most every applica-

tion Q=1.0 [fits to fractional contributions to the oscillations

(Q<1.0) could be instructive in a few instances].   The exponent N

determines the power weighting of the input data for the fit.

Larger values of N accentuate the fits to the data at higher k.

M determines the weighting applied to the sum over shells in the

calculated EXAFS function.   In CARD5 the user determines the

separate weighting of each shell as Qi*K**Ni.  The user should be

aware that an implicit K**1 weighting is applied in the calcula-

tion of each shell and, therefore, should carefully chose the

parameters Qi,Ni in CARD5 to weight each shell the same as the

data.  M should be chosen to compensate for the implicit K**1

weighting of all the shells.  This convention should avoid the

potential of confusion when attempting to balance the weighting

of the data and the calculated EXAFS function.)

EF-7

TESTCASE? N TYPE CHANGE: /<ret>

(The generation of test EXAFS files from the model function,

which is computed in the EXAFIT program, is still in the develop-

mental stage.  If the user requires a test case then one can be

created separately using the TESTFN program.)

SETUP MAP? Y TYPE CHANGE: /<ret>

(A printout of the current setup is produced in a card image

format similar to Lytle's program.)

STORE SETUP? Y TYPE CHANGE: /<ret>

(The current card setup can be stored to disk in one of 5

storage locations.)

After the last <ret> the setup up of CARD 1 is complete and the

input file specified for casename is read in.  A error message

occurs if no case name is specified in CARD 1

NO CASE NAME IN CARD 1

Another error message can occur during a read of the casename

or file not found. In this case the error is terminal to the

EXAFIT program but not before the setup card parameters are

saved.

ABNORMAL TERMINATION: SETUP DATA RECOVERED IN SETUP FILE 6

AFTER EXITING THE EXAFIT PROGRAM, RENAME THIS FILE OR DELETE IT!

EF-8

CARD 2 dialog:

MAXIMUM K-RANGE AVAILABLE

XKMIN:       3.560

XKMAX:      18.356

K-RANGE TO FIT

KMIN:       3.700<ret>

KMAX:      15.500<ret>

(The maximum range of the input data is displayed on the terminal

and should be accurate provided that CARD 1 was setup prior to

CARD2. Spurious results may occur if CARD 1 is not executed first.

The user specifies the fit range which must be a sub-range of the

maximum available range.  A default /<ret> or input of numbers

outside the allowable range produces a default to the maximum

range.)

The dialog for CARD 2 setup is completed.

CARD 3 dialog:

PARAMETERS TO VARY IN THE FIT

SHELL:   1

N ? N    TYPE CHANGE: /<ret>

A ? N    TYPE CHANGE: /<ret>

R ? N    TYPE CHANGE: Y<ret>

SI? N    TYPE CHANGE: /<ret>

B ? N    TYPE CHANGE: /<ret>

C ? N    TYPE CHANGE: /<ret>

P0? N    TYPE CHANGE: /<ret>

P1? N    TYPE CHANGE: /<ret>

P2? N    TYPE CHANGE: /<ret>

P3? N    TYPE CHANGE: /<ret>

G ? N    TYPE CHANGE: /<ret>

H ? N    TYPE CHANGE: /<ret>

E0? N    TYPE CHANGE: Y<ret>

(For multiple shell fits the dialog is repeated.  In this

example the parameters R1 and E0 are to be varied in the fit.

The index on the R is the shell index.  E0 is specified only in

the first shell.  E0 is equivalent to an ENERGY EDGE SHIFT and is

refered to as the "inner potential" in the EXAFS literature.  E0

has the effect of changing the K-scale of input data to match the

K'-scale of the calculated data, where K'= sqrt(K^2-.262467E0).

This completes the dialog for CARD 3 setup.

EF-9

CARD 4 dialog:

TRIAL PARAMETERS

SHELL:  1

N :  0.00000E-01   TYPE CHANGE: 12.0<ret>

A :  0.00000E-01   TYPE CHANGE: 1.0<ret>

R :  0.00000E-01   TYPE CHANGE: 2.556<ret>

SI:  0.00000E-01   TYPE CHANGE: 0.000<ret>

B :  0.00000E-01   TYPE CHANGE: 1.0<ret>

C :  0.00000E-01   TYPE CHANGE: 1.0<ret>

P0:  0.00000E-01   TYPE CHANGE: 1.0<ret>

P1:  0.00000E-01   TYPE CHANCE: 1.0<ret>

P2:  0.00000E-01   TYPE CHANGE: 1.0<ret>

P3:  0.00000E-01   TYPE CHANGE: 1.0<ret>

G :  0.00000E-01   TYPE CHANGE: 0.0<ret>

H :  0.00000E-01   TYPE CHANGE: 1000.0<ret>

E0:  0.00000E-01   TYPE CHANGE: 5.0<ret>

(The dialog is repeated for multiple shell fits.  In this

example the setup of CARD 4 is for a 1-shell fit to the CUFOIL

EXAFS oscillations where the parameters have their respective

meanings in the model EXAFIT equation above.  However, there are

special cases which exist when either A=B=C=1. or P0=P1=P2=P3=1.

When A=B=C=1.0, the parameterized scattering amplitude factor

A/{1+B^2(K-C)^2} is not used in the model calculation, but rather

a reference amplitude which can be specified in CARD 5 and CARD 7.

The scattering amplitude envelope can be obtained from a reference

or standard data file in specified in CARD 5 or obtained from a

table of K,AMPLITUDE pairs entered in CARD 7.  When all of the

phase shift parameters (P's) are 1.0 the scattering phase shift is

determined from a table of K,PHASE SHIFT values entered in CARD 8.

When A,B,C and the P's are not all equal to 1.0 then the respec-

tive CARDS 5,7 and 8 are ignored.)

This ends the setup of CARD 4.

EF-10

CARD 5 dialog:

REFERENCE ENVELOPE DATA

SHELL:  1

NO. OF ENVELOPE POINTS [0 to 100]:   0 TYPE CHANGE: /<ret>

CASE MEMBER:              TYPE CHANGE: CUFOIL.B01<ret>

WEIGHTED BY Q1*K**N1

Q1:      0.000 TYPE CHANGE: .544<ret>

N1:      0.000 TYPE CHANGE: 0.0<ret>

(In this example the reference envelope is obtained from the

CUFOIL.B01 data.  To maintain compatible weightings between the

input data and the calculated function, the weighting of the

reference envelope must be Q1*K**N1.  The prefactor Q1, deter-

mined to be R**2 /N (R=2.556 and N=12), cancels exactly the N/R**2

dependence of the EXAFS amplitude in the reference file.  If no

case member or reference file is specified and A=B=C=1.0 then a

tabular amplitude is assumed.  In this case, the NO. OF ENVELOPE

POINTS must be specified in CARD 5 to be greater than zero.  The

user must then enter the same number of K,AMPLITUDE envelope pairs

in setup CARD 7.)

This ends the setup dialog for CARD 5.

CARD 6 dialog:

TESTCASE NAME:            TYPE CHANGE: /<ret>

(The testcase name determines the file name for writing a test

EXAFS function to disk.  The test EXAFS function is the same

as the calculated function used in the fit.  Presently, the coding

to do this is in the developmental stage, therefore, CARD 6 is not

effective at this time.)

This ends the setup dialog for CARD 6.

EF-11

CARD 7 dialog:

INPUT OF ENVELOPE PAIRS

SHELL:  1

ENVELOPE PAIR 1   0.00000E+00,0.00000E+00

TYPE CHANGES: 3.7,.535<ret>

.

.

.

ENVELOPE PAIR 25  0.00000E+00,0.000000E+00

TYPE CHANGES: 15.5,.0912<ret>

(This dialog may be repeated if multiple shells of tabular

envelopes are specified.  This example illustrates the dialog

for one shell with 25 (maximum of 100) pairs of points specified

in CARD 5.  The envelope pairs are typed in as K,AMPLITUDE. The

EXAFIT program interpolates the envelope points onto the cal-

culated function scale.)

This ends the dialog for CARD 7.

CARD 8 dialog:

INPUT PHASE SHIFT

SHELL 1 HAS   0 PAIRS OF POINTS   TYPE CHANGE: 17<ret>

PHASE SHIFT PAIR:  1    0.00000E+00,0.00000E+00

TYPE CHANGES:  3.7795,24.2716<ret>

.

.

.

PHASE SHIFT PAIR: 17    0.00000E+00,0.00000E+00

TYPE CHANGES: 15.1727,17.8061<ret>

(The dialog may be repeated if multiple shells of tabular phase

shifts are specified.  This example prompts for the number of

of phase shift pairs (maximum of 20) and accepts the typed

K,PHASE SHIFT pairs up to the number of pairs specified.)

The dialog for setup CARD 8 is complete.

EF-12

The normal setup of the cards is now complete.  The user may

specify a card number which has already been set up in order

to check his work or make further changes.  After the user is

satified that all of the setup cards are complete then CARD 0 is

specified. CARD 0 terminates the setup input and passes control

to input the reference files and print or store the setup.  If an

I/O error occurs while attempting to read in the reference file

then the error is usually terminal to EXAFIT program.  The setup

file is usually recovered, prior to the halt, in setup file

number 6 (SAVE FILE FOR ABNORMAL TERMINATIONS).

EF-13

OUTPUT:       The disk and printer output follow the input of the reference

files.   If the storage of the setup file was specified in CARD 1,

then the output of the current setup file to disk is automatially

assumed.  If disk storage was not specified in CARD 1, then the

the user is queried again to confirm that he does not want to save

the setup.  This double check is used to protect the user from

losing the setup after spending a large amount of time to setup a

fit.

AFTER ALL YOUR WORK TO SETUP THIS FIT,

DO YOU WANT TO SAVE THE CARD SETUP PARAMETERS?

(The response of N<ret> passes control to the setup printout)

Y<ret>

****

DO YOU WANT TO LOOK AT A TABLE OF SETUP DATA FILES?

Y<ret>

TABLE OF CURRENT SETUP FILES ON DEFAULT DISK

SETUP FILE 1       CUFOIL.B01

SETUP FILE 2       FEFOIL.B02

SETUP FILE 3       < UNUSED >

SETUP FILE 4       < UNUSED >

SETUP FILE 5       < UNUSED >

SAVE SETUP FILE FOR ABNORMAL TERMINATIONS:

SETUP FILE 6

TYPE NUMBER OF SETUP FILE FOR STORAGE

3<ret>

(In this example the setup file is stored in a previously

unused setup file location and is now available for later recall.)

****

The second output at this stage is the printout of the setup

map, if specified in CARD 1.  The setup map contains all of the

current setup specifications.

EF-14

Sample map:

***** EXAFIT PROGRAM FOR RT-11 *****                                          28-MAY-81

INPUT CARD PARAMETERS FROM SETUP

CARD 1      :MEMBER TO FIT

CUFOIL.B01    1   10   10    5 1.000E-01 1.000E-02 1.000E+00 0.000E+00    0  FFT

CARD 2      :RANGE OF FIT  KMIN,KMAX

3.700E+00 15.500E+00

CARD 3      :PARAMETERS BEING VARIED (T=TRUE,YES   F=FALSE,NO)

/  NUMBER /  A      /   R     / SIGMA   /   B     /   C     /   P0    /   P1    /    P2    /   P3    /   G     /   H     /  E0

FFFFFFFFFFFFFFFFFFFFTFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFT

CARD 4      :VALUES OF THE FIT PARAMETERS

N     1.200000E+00

A     1.000000E+00

R     2.556000E+00

SI    0.000000E-00

B     1.000000E+00

C     1.000000E+00

P0    1.000000E+00

P1    1.000000E+00

P2    1.000000E+00

P3    1.000000E+00

G     0.000000E-00

H     1.000000E+03

E0    5.000000E+00

CARD 5       :ENVELOPE MEMBER INPUT FROM DISK

1    25 CUFOIL.B01  5.440E-01 0.000E-00

CARD 6       :TESTCASE MEMBER

CARD 7       :ENVELOPE PAIRS ENTERED FROM TERMINAL

CARD 8       :PHASE PAIRS ENTERED FROM TERMINAL

SHELL    1     NO. OF PHASE PAIRS   17

3.7795E+00  2.4272E+01    ...

.

.

.

This completes the printout of the setup map.  The iterating stage

begins.

EF-15

ITERATING:    In the EXAFIT program, the iterating begins with the 0th

iteration using the initial unrefined estimates.  The iterating

continues for the maximum number times specified in CARD1 or until

a converged solution or diverging solution occurs.   At the 0th

iteration, the initial guess parameters are displayed on the

printer:

EXAFIT PROGRAM TO FIT ANALYTICAL MODEL FUNCTION TO CUFOIL.B01 EXAFS DATA

NOMINAL PARAMETERS                              SHELL  1                         EDGE ENERGY SHIFT (5.00000E+00)

SHELL  N       A       R      SIGMA**2      B         C          P0         P1       P2         P3         G         H

1  12.00    1.0000  (2.5560)  0.00000    1.0000    1.0000     1.0000     1.0000   1.0000      1.0000    0.000    1000.000

** PARENTHESES INDICATE ESTIMATED VALUES TO BE REFINED BY THIS PROGRAM **

** IMPROVED VALUES ARE TABULATED BELOW **

The refined parameter values are displayed on the printer as well

as the standard deviation of fit after each iteration:

THE BEST RESULTS FOR THIS ITERATION ARE:

THE EDGE SHIFT IS :   (5.26000E+00)

SHELL  N       A        R      SIGMA**2      B         C         P0         P1       P2         P3         G         H

1  12.00    1.0000  (2.5473)  0.00000     1.0000    1.0000    1.0000     1.0000   1.0000      1.0000     0.000   1000.000

THE STANDARD DEVIATION OF THIS FIT IS :  1.3395E-02

EF-16

An additional line is printed for an iteration in which the para-

meters varied by less than the percent specified in CARD 1.  The

iterating is then terminated :

AT THIS ITERATION, ALL PARAMETERS CHANGED BY LESS THAN THE DESIRED PERCENTAGE (  0.0100%)

For an iteration in which a diverging standard deviation occurs,

a single statement is printed signalling a termination to the

fitting :

DIVERGING SOLUTION

At the end of the fitting process a prompt message occurs on the

terminal screen below the final plot.

ANOTHER FIT? (Y/N)

A response of 'Y' returns to the beginning of the EXAFIT program.

This is useful if more iterations are desired.  If a new setup

file is not read in then the values of the parameters from the

last iteration are stored in the cards.

A response of 'N' ends the EXAFIT program.

EF-17

PLOTTING:      The plotting in the EXAFIT program occurs after each completed

iteration.  The input data file is displayed with a solid

line and the model function curve is displayed with a dotted

line.  Appropriate labels are displayed as well as the iteration

number, standard deviation and best damping factor.  After each

iteration a plot is constructed.   The terminal waits for a user

response.  A response of Y<ret> initiates a hard copy of the plot

on the graphics terminal, while any other response ignores the

hard copy and begins the next iteration.  Most terminals have a

buffer in which to store a number of consective responses allow-

ing the user to run the fit iterations unattended.

EF-18

NOTES:         This section is for providing helpful hints and comments.

REMEMBER TO RUN A CARD 1 SETUP BEFORE OTHER CARDS ARE SETUP.

THE CASEMEMBER NAME IN CARD 5 MUST BE DEFAULTED WITH A (/).

IF ONLY <ret> IS STRUCK THEN THE CASEMEMBER NAME IS NOT SPECIFIED

AND A TABULAR AMPLITUDE IN CARD 7 IS ASSUMED.

GENERALLY, THE CURRENT SETUP IS RETRIEVABLE AFTER AN ABNORMAL

TERMINATION OF THE EXAFIT PROGRAM.  THE SETUP IS SAVED IN SETUP

FILE 6 WHEN I/O ERRORS OCCUR DURING INPUT OF THE DATA FILES.

AN ERROR OCCURS IF NOT ENOUGH ROOM IS LEFT ON THE DEFAULT DEVICE

TO WRITE THE SETUP FILES.  THE ERROR IS AN I/O END OF FILE ERROR

WHICH RESULTS IN THE LOSS OF THE SETUP FILE.  THE USER SHOULD BE

EFFICIENT AND TIDY IN THE USE OF DISK SPACE WITH THIS PROGRAM.

ALL INPUT FILES SPECIFIED IN THE SETUP CARDS MUST BE ON THE

DISK AT THE TIME THE CARDS ARE SETUP.

EF-19

INDEX OF EXAFIT DOCUMENTATION

Pages

Introduction                     ..............................  1,2

Operation                        ..............................  2

Hardware                         ..............................  2

Setup                            ..............................  3

Execution                        ..............................  3,4

Cards                            ..............................  5-12

Card 1 dialog             ..............................  5,6,7

Card 2 dialog             ..............................  8

Card 3 dialog             ..............................  8

Card 4 dialog             ..............................  9

Card 5 dialog             ..............................  10

Card 6 dialog             ..............................  10

Card 7 dialog             ..............................  11

Card 8 dialog             ..............................  11

Card 0 cards complete     ..............................  12

Output of setup cards to disk    ..............................  13

Setup map                        ..............................  14

Iterating                        ..............................  15,16

Plotting                         ..............................  17

Notes and comments               ..............................  18

Index                            ..............................  19

PH-1

******************************************************************************

************ DOCUMENTATION FOR EXAFS PHASE TABLE PROGRAM *********************

******************************************************************************

REVISED  11-Nov-82

INTRODUCTION: The PHASE TABLE PROGRAM will allow you to create,inspect, and

alter entries in two types of EXAFS phase function tables which

are used as input for phase fitting (PF) and phase removing (PH)

options in the EXAPLT PROGRAM.

TABLES:       The three types of phase tables are (1) tabular phase tables

(named TEOLEE.PHT after B.K. Teo and P.A. Lee who provided

tabular phase functions from quantum mechanical calculations)

with 17 points on the Teo and Lee grid, (2) TEOLEE.MST, the phase

table with 20 points for multiple scattering modeling, and (2)

parameterized phase tables (named PARAM.PHF) where

phase functions are adequately described by the functional form

Phi(k) = A0 + A1*k + A2*k**2 + A3/k**3 .  For the parameterized

tables, the coefficients A0,A1,A2,A3 are stored and a parameter

E0 which alters the k(wave vector) scale k'= sqrt(k**2-.262467E0).

E0 is identified as the 'inner potential' in the EXAFS literature.

SIZE:         Presently the table TEOLEE.PHT, TEOLEE.MST provides for 300 sets

of tabular entries and the table PARAM.PHF provides for 100 sets

of function entries.  Each tabular or functional set is accessed

by name.  The identifying name is provided by you at the time of

entry but can subsequently be changed along with the numerical

values whenever desired.

PH-2

OPERATION:    The PHASE PROGRAM should be run from the default I/O device. All

three phase tables should also be maintained on the same default

device and if created will automatically exist on the default de-

device.  NOTE OF PRECAUTION:  BACKUP COPIES OF THE CURRENT PHASE

TABLES SHOULD ALWAYS BE MAINTAINED ON ANOTHER DEVICE. The first

question in the program allows you to select the single (17 points

grid) or the multiple scattering (20 points grid) tables. When the

prompt symbol >> occurs on the screen or teletype the phase pro-

gram is waiting for your instructions. A list of valid instuc-

tions (two-letter keywords) is given below with a short descrip-

tion for each option.  A more detailed description of each option

is given further on. All the described commands for the table

TEOLEE.PHT have the same effect on TEOLEE.MST, when multiple

scattering formalism is selected.

OPTIONS:      A list of two-letter responses to the phase program prompt >> :

EX --- Exit from the Phase Program

DT --- Directory of Tabular entries in TEOLEE.PHT

DF --- Directory of Function entries in PARAM.PHF

NT --- New Tabular Phase Table is created

NF --- New Function Phase Table is created

CT --- Check and/or Change existing Tabular Entry

CF --- Check and/or Change existing Function Entry

ET --- New Tabular Entry to TEOLEE.PHT

EF --- New Function Entry to PARAM.PHF

PT --- Printout Directory of Tabular Entries

PF --- Printout Directory of Function Entries

PH-3

DESCRIPTIONS: Detailed description of program operation by option:

>>

EX<ret>

The EX option closes all immediate options and exits

from the phase table program leaving the phase tables at

their current status.

>>

DT<ret>

The DT option displays the directory of the tabular

phase table TEOLEE.PHT by entry name.  The name conven-

tions for this table allow one or two letter symbols for

identification of each entry. Ideally the periodic element

symbol should be used to identify both the absorbing atom

and backscattering atom entries to the phase table. A third

numerical character is used to identify the final state

quantum number associated with the central atom element and

is treated in the directory listing as part of the name

(either 0,1,or 2 is an appropriate entry for indentifying

the central atom final state). If a blank or carriage return

is typed in for the third character then the table entry

will always be assumed to be a backscattering element. An

error message is displayed if the tables are missing from

the default device.

ERROR - PHASE TABLE NOT FOUND ON DISK

An error message is displayed if any (xT) options are chosen

before the DT option.  You must get a directory before pro-

ceding on to other options.

ERROR - YOU HAD BETTER GET A PHASE TABLE DIRECTORY FIRST !

PH-4

>>

DF<ret>

The DF option displays the directory of the phase function

entries in PARAM.PHF by entry name. The naming convention

in this table allows the flexibility of up to 10 characters

typically expressed in a form such as   XXXX-XXXX .  An

example would be    CU1-CU  for a copper absorber, copper

scatterer phase shift.  The 1, which denotes the final state

of the backscattered electrons, is optional to enter in this

table. An error message is displayed if the function table

is not on the default device.

ERROR - PARAMETERIZED PHASE FUNCTION TABLE NOT ON DISK

>>

NT<ret>

The NT option creates a new tabular phase table on the

default device, providing for over 300 entry locations. If

the tabular phase table TEOLEE.PHT already exists on the

default device then the NT option will not supercede the old

one unless the old one is renamed or removed from the de-

fault device.  An error message is displayed if this con-

dition occurs.

ERROR - THEORETICAL PHASE TABLE ALREADY ON DEFAULT DISK

>>

NF<ret>

The NF option is exactly indentical to the above NT

option for the parameterized function table PARAM.PHF.

The new function table provides for 100 entry locations.

The error messsage displayed when this option is executed

and the function table already resides on the default device

is:

ERROR - FUNCTION TABLE ALREADY ON DEFAULT DISK

PH-5

>>

ET<ret>

The ET option allows the creation of a new tabular phase

entry.  Here is a sample dialog:

SPECIFY 1 OR 2 LETTER ELEMENT SYMBOL:   CU<ret>

IF ABSORBER,TYPE FINAL STATE QUANTUM NO. L ,

OTHERWISE <RETURN> :   1<ret>

(L=0,1,2 are acceptable options, or just return assumes

a backscattering phase table entry.)

ENTER THEORETICAL PHASE VALUES AS PROMPTED

TABLE VALUE 1 :   4.2345<ret>

TABLE VALUE 2 :   5.3469<ret>

. . .

TABLE VALUE 17:   4.3513<ret>

>>

Be sure to check for proper entry by listing a directory DT.

>>

EF<ret>

The EF option allows creation of a new parameterized phase

entry.  Sample dialog:

SPECIFY FUNCTION TITLE     XXXX-XXXX  :  CU1-CU<ret>

TYPE  A0,A1,A2,A3,E0   SEPARATED BY COMMAS

2.929,-1.1586,.0245,-84.41,0.<ret>

>>

Be sure to check your entry with a DF directory listing.

PH-6

>>

CT<ret>

The CT option will check or change tabular phase entries.

Sample dialog:

SPECIFY ELEMENT SYMBOL FOR THE SCREEN CHECK :  CU<ret>

IF ABSORBER, TYPE FINAL STATE QUANTUM NO. L ,

OTHERWISE <RETURN> :  2<ret>

CU2    xx.xxxx xx.xxxx xx.xxxx xx.xxxx xx.xxxx xx.xxxx

xx.xxxx xx.xxxx xx.xxxx xx.xxxx xx.xxxx xx.xxxx

xx.xxxx xx.xxxx xx.xxxx xx.xxxx xx.xxxx

(If the entry name you specify is not found in the table

directory then an error message is displayed.

ERROR - CU2   NOT FOUND IN THE PHASE TABLES)

OK ?  N<ret>       (a response of yes assumes a satisfactory

screen check and >> is returned)

TYPE APPROPRIATE CHANGES AS PROMPTED

SPECIFY 1 OR 2 LETTER ELEMENT SYMBOL:  CU<ret>

IF ABSORBER, TYPE FINAL STATE QUANTUM NO. L ,

OTHERWISE <RETURN> :  1<ret>

DEFAULT IS COMMA OR SLASH, THEN <RETURN>

TABLE VALUE 1 :  /<ret>

. . .

TABLE VALUE 17:  4.1586<ret>

CU1    xx.xxxx xx.xxxx xx.xxxx xx.xxxx xx.xxxx xx.xxxx

xx.xxxx xx.xxxx xx.xxxx xx.xxxx xx.xxxx xx.xxxx

xx.xxxx xx.xxxx xx.xxxx xx.xxxx  4.1586

This example altered the quantum number from 2 to 1 and

changed the 17th value of the entry

OK ?  Y<ret>

>>

PH-7

>>

CF<ret>

The CF option will check or change phase function entries.

Sample dialog:

SPECIFY NAME OF FUNCTION FOR SCREEN CHECK :   CU2-CU<ret>

CU2-CU  xxxx.xxxx,xxxx.xxxx,xxxx.xxxx,xxxx.xxxx,xxxx.xxxx

(if the entry is not found in the table then an error

message is displayed

ERROR - CU2-CU      NOT FOUND IN FUNCTION TABLE

OK ?  N<ret>

TYPE NAME CHANGE , DEFAULT IS <RETURN> :  CU1-CU<ret>

ENTER PARAMETER CHANGES WITH PROPER SEPARATING COMMAS

DEFAULT IS NOTHING BETWEEN COMMAS

,,,-85.51,

CU1-CU    xxxx.xxxx xxxx.xxxx xxxx.xxxx  -85.51   xxxx.xxxx

(this example changed the name and the fourth parameter

value of this entry)

OK ?  Y<ret>

>>

>>

PT<ret>

The PT option repeats precisely the DT option but outputs

the directory to the printer.

>>

PF<ret>

The PF option repaets precisely the DF option but outputs

the directory to the printer.

PH-8

NOTES:        This section is for providing helpful hints and comments.

AVOID USING THE SAME NAME FOR TWO ENTRIES. IF THIS HAPPENS, IT IS

ADVISABLE TO RENAME ONE, OTHERWISE THE FIRST ENTRY WILL ALWAYS BE

USED.

ENTRIES CAN'T BE DELETED, BUT OUTDATED ENTRIES CAN BE RENAMED OR

CHANGED IN THE (Cx) OPTIONS. A CONVENIENT WAY TO KEEP TRACK IS TO

RENAME ALL OUTDATED ENTRIES {UNUSED} (THIS MEANS SIMPLY TYPING

'UN'   FOR THE NAME OF A TABULAR ENTRY OR   '<UNUSED>' FOR THE

NAME OF A FUNCTION ENTRY FOR FUNCTIONS) AND ZERO THE VALUES.

THESE UNUSED ENTRIES WILL BECOME AVAILABLE FOR LATER USE ON A

FIRST-COME-FIRST-SERVE BY USING THE (Cx) OPTION AGAIN.

USE OF THE PHASE TABLES FOR STORAGE OF TABULAR OR FUNCTION

AMPLITUDES IS ALSO SUGGESTED.  PROCEED AS IF THESE ENTRIES ARE

PHASE ENTRIES, BUT BE CAREFUL OF NAMING CONVENTIONS WHICH INTER-

FERE WITH PHASE ENTRY NAMES.

REMEMBER TO FREQUENTLY UPDATE THE BACKUP PHASE TABLES WHICH SHOULD

BE MAINTAINED ON ANOTHER DEVICE.

PH-9

INDEX FOR PHASE DOCUMENTATION

Pages

Introduction                     ..................................  1

Tables                           ..................................  1

Size                             ..................................  1

Operation                        ..................................  2

Options                          ..................................  2

Description of options

CF   Check/Change function entry ..................................  6

CT   Check/Change tabular entry  ..................................  7

DF   Directory of function table ..................................  4

DT   Directory of tabular table  ..................................  3

EF   New function entry          ..................................  5

ET   New tabular entry           ..................................  5

EX   Exit                        ..................................  3

NF   Create new function table   ..................................  4

NT   Create new tabular table    ..................................  4

PF   Print function directory    ..................................  7

PT   Print tabular directory     ..................................  7

Notes and comments               ..................................  8

Index                            ..................................  9

CN-1

*************************************************************************

**********  DOCUMENTATION FOR THE CONHLD PROGRAM UTILITY  ***************

*************************************************************************

REVISED 9-FEB-83

INTRODUCTION:  The CONHLD program is a utility program for creating the

CONST.HLD map file.  The program only needs to be executed

once unless the CONST.HLD file is lost or destroyed.  The

newly created file is empty and will build as needed in

the EXAPLT program.  The CONST.HLD file holds or retains

important constants, coefficients, range parameters, and

other relevent information which maps the analysis carried

out in EXAPLT.  This information is stored by EXAPLT invisibly

and can be displayed only by options in EXAPLT or by printing

the header of special format files written in EXAPLT.

INSTRUCTIONS:  Store CONHLD utility and source on auxiliary device until

needed.  To execute, simply type RUN Dev:CONHLD.  The result

is the creation of an empty CONST.HLD file on the system

device.  The file CONST.HLD should be maintained on the system

device for access by EXAPLT.  The CONST.HLD is a Random Access

Unformatted Binary File, therefore do not try to read, list

or print the contents of the file directly.

MC-1

**************************************************************************

*************  DOCUMENTATION FOR THE MCMAST PROGRAM UTILITY  *************

**************************************************************************

REVISED 9-FEB-83

INTRODUCTION:  The MCMAST program is a utility program for creating the

McMaster coefficient table MCMSTR.TBL.  The program only needs

to be executed once unless the MCMSTR.TBL file is lost or

destroyed.  The newly created file is empty and can be filled

as needed in the EXAPLT program.  The MCMSTR.TBL file stores

the x-ray cross section coefficients used in the background

normalization in EXAPLT.  The coefficients can be entered

and changed only through dialogue in the EXAPLT program.

INSTRUCTIONS:  Store MCMAST utility and source on auxiliary device until

needed.  To execute, simply type RUN Dev:MCMAST.  The result

is the creation of an empty MCMSTR.TBL table file on the system

device.  The file MCMSTR.TBL should be maintained on the system

device for access by EXAPLT.  The MCMSTR.TBL is a Random Access

Unformatted Binary File, therefore do not try to read, list

or print the contents of the file directly.

CU-1

*****************************************************************************

******* DOCUMENTATION FOR THE CURSRT GRAPHIC INPUT MODE TEST PROGRAM ********

*****************************************************************************

REVISED 9-FEB-83

INTRODUCTION:  CURSRT is a program to test the graphic crosshair input

mode and graphic output in the PLOT10 Terminal Control System.

The graphic hardware required is at least a Tek 4010-1 Storage

Tube or comparable  emulation.

INSTRUCTIONS:  The CURSRT program should page (clear) the graphic screen,

draw a centered square frame, and present the graphic curser

(cross hairs).  The following list of options exercise the

graphic input and output.  The cross hairs should be located

at a desired point within the framed window before giving a

keyboard response.  The beginning origin is the located at the

lower left hand corner of the window.

OPTIONS:       The curser will await any keyboard response, but will only

perform action on certain keystrokes:

P<ret>  (point)-Draws a point at the curser location and

relocates origin at new curser position.

M<ret>  (move)-Moves origin to new curser position; nothing

drawn.

D<ret>  (draw)-Draws a solid vector from origin to new curser

location and relocates origin at new position.

-<ret>  (dash)-Draws a dashed vector from origin to new curser

location and relocates origin at new position.

N<ret>  (new)-Clears graphic screen and draws new frame.

S<ret>   (stop)-Exits to monitor.

If this test is satisfactory, then the EXAFS programs using the

the graphic input mode should execute properly.