C  ---------------------------------------------------------------------

C----THE FIRST CARD MUST CONTAIN THE NAME OF THE PROBLEM (TITLE).

C    THE NEXT CARD IS USUALLY COMPLETELY BLANK. THE PROGRAM

C    THEN TAKES THE DEFAULT VALUES.. IF YOU WANT TO SPECIFY YOUR

C    OWN PARAMETERS, YOU SHOULD DO THIS AS FOLLOWS0

C  COLUMN  FORMAT  NAME   DEFAULT   MEANING

C    1      A1     MAN      0       PRINTS INSTRUCTIONS,UNLESS MAN=9

C    2      A1     INSTR    0       PRINTS INSTRUCTIONS FOR SPECIAL

C                                   FEATURES IF INSTR=1

C    3      A1     INTENS   0       READS INTENSITIES BESIDES LINE

C                                   POSITIONS IF INTENS=1

C   5- 6    I2     NSYST(1) 0       ORTHORHOMBIC SOLUTION. +1=YES, -1=NO

C   7- 8    I2     NSYST(2) 0       MONOCLINIC SYSTEM.   0 MEANS  INDIFF

C   9-10    I2     NSYST(3) 0       TRICLINIC SYSTEM

C  11-15   F5.2    TOL2    3.0      TOLERANCE ON 2-DIMENSIONAL SEARCH

C  16-20   F5.2    TOL3    4.5      TOLERANCE ON 3-DIMENSIONAL SEARCH

C  21-30   F10.5   WAVEL   1.5405   WAVELENGTH

C  31-32    I2     LINCO    0       NUMBER OF GIVEN LINE COMBINATIONS

C                                   FOR THE 2-DIMENSIONAL SEARCH

C  33-34    I2     K (ENL)  0       ENL0 OUTPUT OF INTERMEDIATE RESULTS(

C  35-36    I2     NQ1      3       NQ1 AND NQ2 DETERMINE THE NUMBERS OF

C  37-38    I2     NQ2      6       LINES TO BE USED IN COMBINATIONS

C                                   FOR FINDING ZONES .

C  39-40    I2     NZ1      6       NZ1 AND NZ2 DO THE SAME FOR

C  41-42    I2     NZ2      6       FINDING LATTICES FROM ZONES.

C  43-44    I2     NR       0       REFINEMENT AND EVALUATION RUN ONLY

C                                   IF NR>0. THE DECK OF COMPLETE

C                                   LATTICES TO BE REFINED (6F10.2)

C                                   WILL FOLLOW THE LINES DECK, ALSO

C                                   TERMINATED BY A CARD WITH ZERO OR

C                                   BLANKS IN COLS 1-10.

C  45-46    I2     INP      5       NUMBER OF THE UNIT ON WHICH TO READ

C                                   THE LINE POSITIONS.

C  47-48    I2     LST      0       LIST OF CALCULATED LINES IF LST=1

C  49-50    I2     J (TEST) 0       TEST OUTPUT IF J>0

C  51-60   F10.5   WMOL    0.0      MOLECULAR WEIGHT.

C  61-70   F10.5   DOBS    0.0      OBSERVED DENSITY.

C  71-80   F10.5   TOLG    3.0      TOLERANCE ON MATCH BETWEEN CALCULATE

C                                   AND OBSERVED Q-VALUES.

C  ---------------------------------------------------------------------