C POWDER DIAGRAMM INDEXING ROUTINE. MADE BY D.G.TAUPIN ACCORDING TO

C HIS PAPER IN J. APPL. CRYST. (1969) P.179.

C

C       THE PURPOSE OF THE PRESENT PROGRAMM IS TO FIND THE UNIT CELL OF

C A NEW SPECIES, KNOWING ITS ACCURATE X-RAY POWDER DIAGRAM. IT ALLOWS

C FOR POSSIBLE EXTRANEOUS LINES ALTHOUGH THIS RESULTS IN LESS RELIABI-

C LITY AND IN MORE COMPUTING TIME SPENT.

C       THIS ROUITNE IS NOT ADEQUATE FOR CHEMICAL ANALYSIS OF MIXTURES

C CONTAINING SEVERAL SPECIES WHOSE X-RAY PATTERNS ARE KNOWN; ONE SHOULD

C THEN USE POWDER-DATA-FILE SEARCHING ROUTINES LIKE THE JOHNSON-VAND

C PROGRAM DELIVERED BY A.S.T.M..

C

C ......................................................................

C

C HOW TO USE THIS PROGRAM

C FIRST CARD

C=============

C COL 1-12   OPTION LETTERS ( IN ANY ORDER )

C            'Q' MEANS THAT GIVEN LINE DATA ARE EXPRESSED AS

C                Q=1/D**2  RATHER THAN  D(HKL) WHICH IS THE DEFAULT

C                OPTION ; IN THIS CASE THE GIVEN ERROR ESTIMATION RELATE

C                TO 'Q' INSTEAD OF 'D' ;

C                (IF VALUES ARE .GT.1 THEN THEY ARE ASSUMED TO BE 10000/

C            'A' MEANS THAT GIVEN LINE DATA ARE MEASURED ANGLES;

C                ( NORMALLY 2-THETA )

C            'A1' MEANS THAT THETA ANGLES ARE TO BE READ

C            'A4' MEANS THAT 4-THETA ANGLES ARE TO BE READ ;

C

C            IN ALL THE ABOVE CASES, THE STATED ERROR IS AN ABSOLUTE

C            ERROR CORRESPONDING TO THE VALUE ACTUALLY READ.

C

C            'V' MEANS THAT CELLS ARE ACCEPTED ONLY IF THEY CONTAIN AN A

C                PROXIMATELY INTEGER NUMBER OF ASYMETRICAL UNITS OR

C                MOLECULES ; IF THIS OPTION IS TAKEN, THE SECOND CARD

C                MUST CONTAIN ADDITIONAL VALUES FOR THE

C                       - MOLECULAR WEIGHT,

C                       - DENSITY,

C                       - ESTIMATED ERROR ON THAT DENSITY ;

C            'X' OVERRIDES THE NORMAL SELECTION CRITERIUM WHICH RESULTS

C                IN PRINTING THE ONLY CELLS WHOSE VOLUME DOES NOT EXCEED

C                25 ( MORE THAN THE SMALLEST ONE ALREADY FOUND, AND IS

C                ONE OF THE 10 SMALLEST CELLS FOUND ; IF 'X' APPEARS,

C                THE PROGRAMM WILL PRINT ALL FOUND CELLS WHOSE VOLUME IS

C                LESS THAN THE VALUE STATED IN SECOND CARD ;

C            'C','T','H','O','M','3' MEAN THAT TRIALS MUST BE MADE FOR

C                CUBIC, TETRAGONAL, HEXAGONAL, ORTHORHOMBIC, MONOCLINIC,

C                OR TRICLINIC CELLS RESPECTIVELY ; THESE OPTIONS ARE

C                CUMULATIVE AND IF NONE OF THEM APPEARS THEY ARE ASSUMED

C                TO BE ALL PRESENT ;

C            'P' MEANS THAT THE DATA CORRESPONDING TO THE BEST CELLS

C                WILL ALSO BE PUNCHED ON CARDS ;

C            'L' PROVIDES A SPECIAL OUTPUT ON UNIT 'ILEAST' FOR FURTHER

C                LEAST SQUARE REFINEMENT (APPLEMAN'S PROGRAM) WITH ADE

C                QUATE FORMAT;

C            'Z' IS ESSENTIALLY A DEBUGGING OPTION WHICH PROVIDES ADDI

C                TIONAL LOGGING AND A PRINTOUT OF THE INDEX TABLES AT

C                THE BEGINNING.

C            'F' (FAST) SUPPRESS TRACE PRINTING OF CELLS REJECTED AS

C                UNINDEXABLE, ETC.

C            'G' (YET FASTER) SUPPRESS TRACE PRINTING OF CELLS REJECTED

C                WHEN 'FIGURE OF MERIT' FALLS BELOW PRESET VALUE

C       ALSO 'S' PURPOSE UNCLEAR (AT LEAST TO R.SHIRLEY) - SEE LINE NO.

C                TAUP74-1.439 IN S/R CONTRL

C

C  ** LOCAL DEFAULTS **   (UNIVERSITY OF SURREY, ENGLAND)

C            'F', 'G', 'X'

C            ('F' @ 'G' ARE OVER-RIDDEN IF 'Z' IS SELECTED)

C

C COL 13-72   ANY ALPHANUMERIC TITLE WHICH WILL BE PRINTED AT THE TOP OF

C             EACH PAGE ;

C

C SECOND CARD

C==============

C ALL THESE VALUES ARE IN |FREE FORMAT|, I.E. THEY MAY BE WRITTEN EITHER

C AS INTEGERS OR REAL VALUES ( WITHOUT DECIMAL EXPONENT HOWEVER ), SEPA

C RATED BY COMMAS; BLANK ARE DISREGARDED IN THAT CARD AND LETTERS 'I'

C AND 'O' ARE UNDERSTOOD AS FIGURES '1' AND '0'.

C THESE VALUES ARE RESPECTIVELY

C       - NUMBER OF GIVEN EXPERIMENTAL LINES

C       - MAXIMUM NUMBER OF EXTRANEOUS LINES

C       - MAXIMUM DEGENERESCENCE FACTOR   MAXIMUM NUMBER OF ADDITIONAL

C         LINES TO BE USED WHEN THE FIRST ONES LEAD TO A DEGENERATE EQUA

C         TION SYSTEM FOR CELL PARAMETER DETERMINATION (A GOOD VALUE IS

C         2 OR 3)

C       - UPPER LIMIT FOR H**2+K**2+L**2 FOR THE FIRST LINE ( THIS

C         VALUE IS USED TO COMPUTE THE UPPER LIMIT FOR H**2+K**2+L**2 OF

C         THE FOLLOWING LINES, ASSUMING THEY ARE PROPORTIONAL TO

C         Q=1/D**2 ( THIS IS OBVIOUSLY A ROUGHLY APPROXIMATE EXTRAPOLA

C         TION )

C       - MAXIMUM TIME LIMIT ( MINUTES ); WHEN THIS LIMIT IS EXCEEDED

C         THE PROGRAMM WILL SUSPEND ITSELF AND WRITE 2 OR MORE RESTART

C         ING RECORDS; THIS LIMIT SHOULD BE SMALLER THAN THAT STATED IN

C         THE 'JOB' CARD TO PREVENT IT FROM BEING KILLED BY THE

C         OPERATING SYSTEM.

C       - MAXIMUM VOLUME OF THE UNIT CELL (ANGSTROEM**3)

C       - THE MINIMAL FIGURE OF MERIT (SEE DE-WOLFF'S PAPER IN JOURNAL

C         OF APPLIED CRISTALLOGRAPHY - 1968); USUAL VALUE = 4; HIGHER

C         VALUES MAY BE STATED IF EXPERIMENTAL MEASUREMENTS ARE GOO;.

C    WHEN OPTION 'V' IS PRESENT

C       - MOLECULAR MASS ( OR MASS OF THE ASYMETRIC UNIT )

C       - DENSITY ( GRAMS/CM**3 )

C       - RELATIVE ESTIMATED ERROR ON THAT DENSITY

C

C=============# NOTE THAT YOU MAY LEAVE ALL OR PART OF THESE TWO FIRST

C CARDS BLANK; THEN THE PROGRAM WILL USE DEFAULT VALUES WHICH ARE

C - ALL SYSTEM TESTED;

C - LINES WILL BE COUNTED TO FIND THEIR EFFECTIVE NUMBER;

C - NO EXTRANEOUS LINES ALLOWED;

C - DEGENERESCENCE FACTOR EQUAL TO 2 ;

C - H**2+K**2+L**2 LIMITED TO 6 FOR THE FIRST LINE;

C - NO TIME LIMIT, THEN NO RESTART ;

C - MAXIMUM VOLUME = 1000 ANGSTROEM**3 ;

C - FIGURE OF MERIT EQUAL TO 4 ;

C

C FOLLOWING CARDS

C==================

C ONE CARD FOR EACH LINE; FORMAT IS (F10.0,F10.0,1X,A1,3I3) ;

C VALUES ARE RESPECTIVELY

C       - D(HKL) IN ANGSTROEMS OR 1/D(HKL)**2 IF 'Q' OPTION;

C       - ABSOLUTE ESTIMATED ERROR ON D(HKL) OR ON Q=1/D**2 IF 'Q' OPT;

C         IF THIS VALUE IS BLANK, THEN EXTRAPOLATION IS MADE ASSUMING

C         THE ERROR ON 1/D IS CONSTANT ;

C       - COL 22   ANY NON BLANK CHARACTER MEANS THAT THIS LINE MAY

C         BE EXTRANEOUS; THIS ALLOWS THE PROGRAMM TO CONTINUE A TRIAL

C         EVEN IF IT WAS NOT ABLE TO ASSIGN INDICES TO THAT LINE; IF

C         ALL LINES HAVE BLANK CHARACTER IN COL-22, ALTHOUGH THE

C         MAXIMUM NUMBER OF EXTRANEOUS LINES IS NON-ZERO, THEN ALL

C         LINES ARE CONSIDERED AS POSSIBLY EXTRANEOUS.

C       - COL 23-25,26-28,29-31   VALUES OF H, K, L, FOR THAT LINE,

C         IF THEY ARE KNOWN; ONLY 10 LINES ARE ALLOWED TO HAVE H,K,L

C         INDICATIONS;

C