DOCUMENTATION FOR PROGRAM LOGIC

Program LOGIC is used to search the ICDD-JCPDS database for materials with

selected characteristics.

The ICDD-JCPDS database contains approximately 60,000 sets of summarized

diffraction patterns. The database is copyright by the International

Centre for Diffraction Data and may be used only by license.

This program maintains a working subset of the full database, referred

to as the "current table" where the contents of the table may be

increased or decreased according to logical criteria  set by the user.

There are five basic categories of instructions that may be given to

LOGIC.

Commands READ and WRITE are used to read in or write out a stored table;

commands TABLES, BROWSE, and INDEX are used to examine the entries

in the current table;

commands ADD, MATCH and CULL modify the current list by applying logical

criteria;

command WAVELENGTH controls whether the program will use d-spaces or

two-theta angles for input and output; and command DELETED controls

usage of "deleted" patterns.

commands DCL, FILE, HELP and EXIT are used for program control.

The following element symbols are defined within program LOGIC:

H, He, Li, Be, B, C, N, O, F, Ne, Na, Mg, Al, Si, P, S, Cl,

Ar, K, Ca, Sc, Ti, V, Cr, Mn, Fe, Co, Ni, Cu, Zn, Ga, Ge,

As, Se, Br, Kr,	Rb, Sr, Y, Zr, Nb, Mo, Tc, Ru, Rh, Pd, Ag,

Cd, In,	Sn, Sb, Te, I, Xe, Cs, Ba, La, Ce, Pr, Nd, Pm, Sm,

Eu, Gd, Tb, Dy, Ho, Er, Tm, Yb, Lu, Hf, Ta, W, Re, Os, Ir,

Pt, Au, Hg, Tl, Pb, Bi, Po, At, Rn, Fr, Ra, Ac, Th, Pa, U,

Np, Pu, Am, Cm, Bk, Cf, Es, Fm, Md, No, Lr

LN    Lanthanides (La - Lu)

AD    Actinides (Ac - Lr)

1a    period 1 (H,Li,Na,K,Rb,Cs,Fr)

2a    period 2 (Be,Mg,Ca,Sr,Ba,Ra)

3a    period 3 (B,Al,Ga,In,Tl)

4a    period 4 (C,Si,Ge,Sn,Pb)

5a    period 5 (N,P,As,Sb,Bi)

6a    period 6 (O,S,Se,Te,Po)

7a    period 7 (H,F,Cl,Br,I,At)

8a    period 8 (He,Ne,Ar,Kr,Xe,Rn)

1b    transition metals period 9 (Cu,Ag,Au)

2b    transition metals period 10 (Zn,Cd,Hg)

3b    transition metals period 1 (Sc,Y,La,Ac)

4b    transition metals period 2 (Ti,Zr,Hf)

5b    transition metals period 3 (V,Nb,Ta)

6b    transition metals period 4 (Cr,Mo,W)

7b    transition metals period 5 (Mn,Tc,Re)

8b    transition metals period 6-8 (Fe,Co,Ni,Ru,Rh,Pd,Os,Ir,Pt)

===> COMMAND  ADD

Command ADD increases the number of entries in the current table using

a logical .OR. operation to add all entries which match a specified

logical criterion. Command OR is completely equivalent to command ADD.

There are four possible logical criteria that may be established:

/ELEMENTS A list of entries is specified by a partial or

complete elemental composition

/CARD     A list of entries is specified by ICDD-JCPDS number

/PEAKS    A list of entries is specified by positions of the major

peaks in the pattern.

/COUNT    A list of entries is specified by the number of different

elements in the chemical formula.

Qualifiers may be specified in only certain combinations. Square brackets

([ ]) indicate parameters that are allowed but are not required.

For example, if /ELEMENTS/OPTIONAL is used, /EXACT is assumed,

and need not be specified explicitly.

Angle brackets (< >) indicate that only one of the enclosed

parameters parameters may be chosen. For example, for /PEAKS/OR

either /DSPACE or /WAVELENGTH may be used, but not both.

Curly brackets ({ }) indicate that more than one of the the enclosed

parameters parameters may be chosen, but at least one must be chosen.

For example, either /OPTIONAL or /I_SPAN or both may be specified

with /ELEMENTS but one of the two must be specified, unless /OR or

/AND or /EXACT is used.

ADD {/ELEMENTS= /E_SPAN=} /OR

ADD {/ELEMENTS= /E_SPAN=} /AND

ADD {/ELEMENTS= /E_SPAN=} /EXACT

ADD {/ELEMENTS= /E_SPAN=} {/OPTIONAL= /O_SPAN=} [/EXACT]

ADD/CARDS

ADD/COUNT=

ADD/PEAKS=/OR/FAST  [/ERROR=] [/LEVEL=] </DSPACE /WAVELENGTH=]>

ADD/PEAKS=/AND/FAST [/ERROR=] [/LEVEL=] </DSPACE /WAVELENGTH=]>

ADD/PEAKS=/OR/EXACT/ERROR=  [/LEVEL=] </DSPACE /WAVELENGTH]>

ADD/PEAKS=/AND/EXACT/ERROR= [/LEVEL=] </DSPACE /WAVELENGTH]>

The /ELEMENTS (or /E_SPAN), /CARD or /PEAKS qualifier

should be specified before other all other qualifiers.

---  Qualifiers

/ELEMENTS=required_list

/ELEMENTS

/E_SPAN=req_range(s)

Use this options to specify a list of elements which must be present for

the logical criterion to be matched. The list of elements may be used

in one of four different ways:

/OR

An entry with a composition containing any one of the

specified elements is a match.

/AND

An entry with a composition containing all of the

specified elements is a match.

/EXACT

An entry with a composition containing all of the

specified elements and no others is a match.

/OPTIONAL (and/or /O_SPAN)

An entry with a composition containing all of the

specified elements and any or none of the optional

elements, but no others no others is a match.

Use /E_SPAN to specify a range of elements which must be present for

the logical criterion to be matched. This option is equivalent to

specifying the elements individually in the required_list using

/ELEMENTS.

Examples of the use of these qualifiers are:

/ELEMENTS=SI

/elem=(si,o,H)

/E_SPAN=(H,BE)

/E_SPAN=(Be,H) (same as previous)

/ELEMENTS=(H,HE,LI,BE) (same as previous)

/E_SPAN=(H,BE,C,F) (specifies two ranges: from H to Be and

from C to F.

Note that capitalization is ignored and the elements must be surrounded

by parentheses and separated by commas, if more than one element is

specified. For the /E_SPAN qualifier, element ranges may be

specified in either ascending or descending order.

To specify more than one range, specify more than one pair of endpoints.

If no elements are specified with /ELEMENTS, and /E_SPAN is not

used, the program will prompt for elements.

The following qualifiers may not be used with /ELEMENTS or /E_SPAN:

/CARDS, /PEAKS, /FAST, /ERROR, or /LEVEL.

The /ELEMENTS or /E_SPAN qualifier should be specified before

other all other qualifiers.

/COUNT=elm_count

/COUNT=(c_min,c_max)

/COUNT

Use this qualifier to select entries using the number of unique elements

in the chemical formula. The qualifier may be used three ways, if a

single value is specified, for example /COUNT=2, then entries are

selected with that single number of elements. If two values are used,

for example /COUNT=(2,4) the two numbers represent a range of values,

in this case 2, 3 or 4 elements are allowed. If no value is specified,

the computer will prompt for one or more values.

Note that Cr2O3 . xH2O will be counted as having 3 elements in the

formula.

No other qualifiers are allowed with /COUNT.

/CARDS=card_list

/CARDS

Use this qualifier to select entries using their ICDD-JCPDS entry number.

The entry number is always a five or six digit number consisting

of the "old" entry number plus 10,000 times the set number. For

example, entry 413 in set 7 would be coded as:

/CARD=70413 or

/CARD=070413

To specify more than one entry, separate the list with commas and

enclose the list in parentheses:

/CARD=(070413,10001)

No other qualifiers are allowed with use of the /CARDS qualifier.

/PEAKS=peak_list

/PEAKS

Use this option to select entries based on the location of the

intense peaks in the pattern. The peak positions must be input

as either two-theta angles or as d-spaces depending on the

current wavelength setting (see command WAVELENGTH). The

wavelength setting may be overridden using the /WAVELENGTH

or the /DSPACE qualifiers.

All entries with intense peaks (the intensity threshold is set by

the /LEVEL option) within some region around input peak

position will be selected. The actual window depends on the

value(s) used on the /ERROR option.  If more than one peak

is input, the peaks should be separated by  commas and enclosed

in parentheses.  The list of peaks may be used in one of two

different ways:

/OR  An entry with a composition containing any one of the

specified peaks is a match.

/AND An entry with a composition containing all of the

specified peaks is a match.

If no peaks are input, the program will prompt for a list of peaks.

The following qualifiers may not be used with /PEAKS:

/CARDS, /ELEMENTS, /E_SPAN, /COUNT,

/OPTIONAL, or /O_SPAN.

Specify /PEAKS as the first qualifier on the line.

/ERROR=error_list

The error_list specifies an allowed range for each peak.

The range is used in one of two ways, depending on mode used by

the program. If the mode is /EXACT, the error range is used

exactly as specified. For example, If

/PEAK=15.5/ERROR=0.1/EXACT

is used, peaks at locations between 15.4 and 15.6 will be

selected. The other mode, /FAST, may be considerably more rapid

than /EXACT. In /FAST mode, the range specified with /ERROR

is expanded to match spacing of records in the peak index files.

For example, if the peak index records cover the ranges, 15.15-15.3,

15.3-15.45, 15.45-15.6, etc., then if

/PEAK=15.5/ERROR=0.1/FAST

is used, then the error range will be expanded to select peaks

between 15.3 and 15.6.

An error value may be specified for each peak, but if the number

of values in the error_list is less than the number of peaks in

the peak_list, the last error value will be used for all

remaining peaks. If the /ERROR parameter is not used, the

program will prompt for a single error value for all peaks.

The error value should be specified in the same units (d-space

or two-theta) as the peak positions.

The /PEAKS qualifier must precede this qualifier.

The following qualifiers may not be used with /ERROR:

/CARDS, /ELEMENTS, /E_SPAN, /COUNT,

/OPTIONAL, or /O_SPAN.

/DSPACE

Using this qualifier overrides the current wavelength setting and forces

peaks to be input as d-spaces. This qualifier cannot be used with

/WAVELENGTH and will be ignored if used with a qualifier other than

/PEAKS.

/WAVELENGTH=wave_length

Using this qualifier overrides the current wavelength setting and forces

peaks to be input as two-theta angles at wave_lengthAngstrom. This qualifier

cannot be used with /DSPACE and will be ignored if used with a qualifier

other than /PEAKS.

/LEVEL=peak_level

Specifies a relative intensity threshold for peak positions specified

with the /PEAKS qualifier. The threshold value is used slightly

differently in /FAST mode versus /EXACT mode. In /EXACT mode,

the value specifies the exact intensity threshold (in percent relative

intensity). For example if /PEAKS=20.5/ERR=0.1/LEVEL=75/DSPACE/EXACT

is specified, entries with peaks having d-spaces from 20.4 to 20.6 Angstrom

with relative intensities of 75% or greater will be located.

In /FAST mode, the threshold value specifies a peak index to

be searched. If there is no peak index at this value, a peak index

with a lower threshold is used (at 10 % increments). Thus in the

example /PEAKS=20.5/ERR=0.1/LEVEL=75/DSPACE/FAST, the error range is

opened (see /ERROR) and the intensity threshold is also possibly

lowered, so that potentially more entries will be selected than in

/EXACT mode. It is also possible to specify /LEVEL=3s in /FAST mode.

This specifies an index of the three strongest peaks in each entry.

The /PEAKS qualifier must precede this qualifier.

The following qualifiers may not be used with /LEVEL:

/CARDS, /ELEMENTS, /E_SPAN, /COUNT

/OPTIONAL, or /O_SPAN.

/EXACT

Use /EXACT with /ELEMENTS to specify that only entries

containing all of the specified elements and no others

match as the specified logical criterion. It is possible to widen

this criterion so that certain elements are allowed, but not required,

using the /OPTIONAL and/or /O_SPAN qualifiers.

Use /EXACT with /PEAKS to specify that the error range specified

for peak positions should not be extended, but instead be used as given.

(See the description given for /ERROR.)

The following qualifiers may not be used with /EXACT and /ELEMENTS:

/CARDS, /PEAKS, /OR, /AND, /ERROR, /FAST,

/COUNT or /LEVEL.

The following qualifiers may not be used with /EXACT and /PEAKS:

/CARDS, /ELEMENTS, /E_SPAN, /FAST, /COUNT

/OPTIONAL, or /O_SPAN.

/FAST

Use /FAST with /PEAKS to specify that the error range specified

for peak positions may be extended for most efficient searching.

(See the description given for /ERROR.)

The following qualifiers may not be used with /FAST:

/CARDS, /ELEMENTS, /E_SPAN, /OPTIONAL, /EXACT,

/COUNT or /O_SPAN.

/OR

Use /OR with either /PEAKS or /ELEMENTS to specify that any entry

containing the specified element or peak is a match for the specified

logical criterion.

The following qualifiers may not be used with /OR:

/CARDS, /AND, /COUNT, /OPTIONAL, or /O_SPAN.

Also, if /ELEMENTS and /OR are used, /EXACT cannot be used.

/AND

Use /AND with either /PEAKS or /ELEMENTS to specify that only entries

containing all of the specified elements or peaks match for the specified

logical criterion.

The following qualifiers may not be used with /AND:

/CARDS, /OR, /COUNT, /OPTIONAL, or /O_SPAN.

Also, if /ELEMENTS and /AND are used, /EXACT cannot be used.

/OPTIONAL=optional_list

/O_SPAN=opt_range(s)

Use /OPTIONAL or /O_SPAN with /ELEMENTS to specify that only

entries containing all of the required elements specified in the

required_list (see /ELEMENTS and /E_SPAN) and optionally those

in the optional_list, but no others, as the specified logical

criterion. /O_SPAN is equivalent to /OPTIONAL except that ranges

of elements may be specified.

Examples of the use of this qualifier are:

/OPTIONAL=SI

/ig=(si,o,H)

/O_SPAN=(H,BE)

/O_SPAN=(BE,H) (equivalent to previous)

/OPTIONAL=(H,HE,LI,BE) (equivalent to previous)

/O_SPAN=(H,BE,C,F) specifies two ranges: from H to Be and

from C to F.

Note that capitalization is ignored and the elements must be surrounded

by parentheses and separated by commas, if more than one element is

specified.

Ranges may be specified in either ascending or descending order.

To specify more than one range, specify more than one pair of endpoints.

The following qualifiers may not be used with /OPTIONAL or /O_SPAN:

/CARDS, /PEAKS, /FAST, /OR, /AND,

/ERROR, /COUNT, or /LEVEL.

Qualifier /EXACT is assumed to be present if /OPTIONAL or /O_SPAN

is used and thus is optional. /O_SPAN may be used with /OPTIONAL.

===> COMMAND  BROWSE

Use this function to examine tables of angles or d-spaces for all entries

in the "current list" from a terminal. Entries are displayed one at a

time; it is possible to page backwards or forwards in the list and to

print selected entries. If the full ICDD-JCPDS (PDF-2) database is on-line,

the entire ICDD-JCPDS entry may be displayed or printed.

After entering the BROWSE command, a prompt line is displayed, which

shows the total number of entries in the current list and the relative

position of the current entry within the list. Also displayed is a

list of available commands.  The following commands are defined:

+n_p moves forward in the list by n_p entries;

-n_p moves backward by n_p entries;

>n_p displays the n_p-th entry in the list;

[crd displays ICDD-JCPDS entry crd, or the closest entry in the

list by number;

(blank line) displays the next entry;

P    prints the chemical information and D/I data (PDF-1 data)

for the current entry and advances to the next entry;

C    culls (removes) the current entry from the current table

and advances to the next entry;

D    displays all the information for the current entry

in the full ICDD-JCPDS (PDF-2) database;

A    prints All the information for the current entry

and advances to the next entry;

G    graphically displays one or more entries, starting from

the current entry;

^Z   or advancing past the last entry terminates BROWSE mode.

The command letter or symbol, (+, > or P, for example) must be

the first letter on the line. Commands which are not valid are not

given in the list of options.

Commands D and A both require the PDF-2 database either on disk or on

CD-ROM. If the PDF-2 database is unavailable, these options will not be

listed.

The graph command, G, is only available on graphics-compatible terminals.

This command can be used to display experimental data as well as ICDD-JCPDS

entries. Each time the command is called, one can input a data or peak file

to the program and this data will be displayed as well as the selected

entries. A wide variety of peak and data files may be read

Of particular use will be SEARCH temporary files, which may

be read directly at this point by LOGIC and may then then be plotted.

===> COMMAND  COMMENT

The program records a summary of the logical steps (commands that

modify the contents of the current table). Use this command to

place a comment line into the list of logical steps. The comment

may be placed on the same line as the COMMENT command, or will be

prompted.

---  Parameter

comment_line

A single line of comments may be placed after the COMMENT command.

Lowercase letters will be translated to uppercase if the

comment_line is placed on the same line, rather than entered at

the prompt.

===> COMMAND  CULL

Command CULL decreases the number of entries in the current table using

a logical .AND. .NOT. operation to remove all entries which match a

specified logical criterion. Command NAND is completely equivalent to

command CULL.

There are five possible logical criteria that may be established:

/ELEMENTS A list of entries is specified by a partial or

complete elemental composition

/CARD     A list of entries is specified by number

/PEAKS    A list of entries is specified by positions of the

major peaks in the pattern.

/NAME     A list of entries is specified by chemical/mineral name

/COUNT    A list of entries is specified by the number of different

elements in the chemical formula.

Qualifiers may be specified in only certain combinations. Square brackets

([ ]) indicate parameters that are allowed but are not required.

For example, if /ELEMENTS/OPTIONAL is used, /EXACT is assumed,

and need not be specified explicitly.

Angle brackets (< >) indicate that only one of the enclosed

parameters parameters may be chosen. For example, for /PEAKS/OR

either /DSPACE or /WAVELENGTH may be used, but not both.

Curly brackets ({ }) indicate that more than one of the the enclosed

parameters parameters may be chosen, but at least one must be chosen.

For example, either /OPTIONAL or /I_SPAN or both may be specified

with /NOELEMENTS but one of the two should be specified.

CULL {/ELEMENTS= /E_SPAN=} /OR

CULL {/ELEMENTS= /E_SPAN=} /AND

CULL {/ELEMENTS= /E_SPAN=} /EXACT

CULL {/ELEMENTS= /E_SPAN=} {/OPTIONAL= /O_SPAN=} [/EXACT]

CULL/NOELEMENTS {/OPTIONAL= /O_SPAN=} [/EXACT]

CULL/CARDS=

CULL/NAME=

CULL/COUNT=

CULL/PEAKS=/OR/FAST  [/ERROR=] [/LEVEL=] </DSPACE /WAVELENGTH=]>

CULL/PEAKS=/AND/FAST [/ERROR=] [/LEVEL=] </DSPACE /WAVELENGTH=]>

CULL/PEAKS=/OR/EXACT/ERROR=  [/LEVEL=] </DSPACE /WAVELENGTH=]>

CULL/PEAKS=/AND/EXACT/ERROR= [/LEVEL=] </DSPACE /WAVELENGTH=]>

The /ELEMENTS (or /E_SPAN), /CARD, /NAME or /PEAKS qualifier

should be specified before other all other qualifiers.

---  Qualifiers

/ELEMENTS=required_list

/ELEMENTS

/NOELEMENTS

/E_SPAN=req_range(s)

Use this options to specify a list of elements which must be present for

the logical criterion to be matched. The list of elements may be used

in one of four different ways:

/OR

An entry with a composition containing any one of the

specified elements is a match.

/AND

An entry with a composition containing all of the

specified elements is a match.

/EXACT

An entry with a composition containing all of the

specified elements and no others is a match.

/OPTIONAL (and/or /O_SPAN)

An entry with a composition containing all of the

specified elements and any or none of the optional

elements, but no others no others is a match.

Use /E_SPAN to specify a range of elements which must be present for

the logical criterion to be matched. This option is equivalent to

specifying the elements individually in the required_list using

/ELEMENTS.

The /NOELEMENTS option is used if there are no required elements,

but optional elements will be specified using the /OPTIONAL or

/O_SPAN options. Thus, entries will be removed that have any

combination of the optional elements and no others. Use of this

option will remove all entries with unassigned chemical formulas,

as would be performed by a CULL/COUNT=0 command.

Examples of the use of these qualifiers are:

/ELEMENTS=SI

/elem=(si,o,H)

/E_SPAN=(H,BE)

/E_SPAN=(Be,H) (same as previous)

/ELEMENTS=(H,HE,LI,BE) (same as previous)

/E_SPAN=(H,BE,C,F) (specifies two ranges: from H to Be and

from C to F.

Note that capitalization is ignored and the elements must be surrounded

by parentheses and separated by commas, if more than one element is

specified. For the /E_SPAN qualifier, element ranges may be

specified in either ascending or descending order.

To specify more than one range, specify more than one pair of endpoints.

If no elements are specified with /ELEMENTS, and /E_SPAN is not

used, the program will prompt for elements.

The following qualifiers may not be used with /ELEMENTS or /E_SPAN:

/CARDS, /NAME, /PEAKS, /FAST, /ERROR, or /LEVEL.

The /ELEMENTS or /E_SPAN qualifier should be specified before

other all other qualifiers.

/COUNT=elm_count

/COUNT=(c_min,c_max)

/COUNT

Use this qualifier to select entries using the number of unique elements

in the chemical formula. The qualifier may be used three ways, if a

single value is specified, for example /COUNT=2, then entries are

selected with that single number of elements. If two values are used,

for example /COUNT=(2,4) the two numbers represent a range of values,

in this case 2, 3 or 4 elements are allowed. If no value is specified,

the computer will prompt for one or more values.

Note that Cr2O3 . xH2O will be counted as having 3 elements in the

formula.

No other qualifiers are allowed with /COUNT.

/CARDS=card_list

/CARDS

Use this qualifier to select entries using their ICDD-JCPDS entry number.

The entry number is always a five or six digit number consisting

of the "old" entry number plus 10,000 times the set number. For

example, entry 413 in set 7 would be coded as:

/CARD=70413 or

/CARD=070413

To specify more than one entry, separate the list with commas and

enclose the list in parentheses:

/CARD=(070413,10001)

No other qualifiers are allowed with use of the /CARDS qualifier.

/NAME=compound_name

/NAME=mineral_name

A fragment of a chemical or mineral name to be matched. The name

listed on an entry must match the compound_name fragment letter

for letter (lowercase letters will be converted to uppercase). If

spaces are to be included in the fragment, the fragment should be

enclosed in quotation marks ("). For example,

/NAME=silicon

/NAME="silicon oxide"

No other qualifiers may be used with /NAME.

/PEAKS=peak_list

/PEAKS

Use this option to select entries based on the location of the

intense peaks in the pattern. The peak positions must be input

as either two-theta angles or as d-spaces depending on the

current wavelength setting (see command WAVELENGTH). The

wavelength setting may be overridden using the /WAVELENGTH

or the /DSPACE qualifiers.

All entries with intense peaks (the intensity threshold is set by

the /LEVEL option) within some region around input peak

position will be selected. The actual window depends on the

value(s) used on the /ERROR option.  If more than one peak

is input, the peaks should be separated by  commas and enclosed

in parentheses.  The list of peaks may be used in one of two

different ways:

/OR  An entry with a composition containing any one of the

specified peaks is a match.

/AND An entry with a composition containing all of the

specified peaks is a match.

If no peaks are input, the program will prompt for a list of peaks.

The following qualifiers may not be used with /PEAKS:

/CARDS, /NAME, /ELEMENTS, /E_SPAN, /COUNT,

/OPTIONAL, or /O_SPAN.

Specify /PEAKS as the first qualifier on the line.

/ERROR=error_list

The error_list specifies an allowed range for each peak.

The range is used in one of two ways, depending on mode used by

the program. If the mode is /EXACT, the error range is used

exactly as specified. For example, If

/PEAK=15.5/ERROR=0.1/EXACT

is used, peaks at locations between 15.4 and 15.6 will be

selected. The other mode, /FAST, may be considerably more rapid

than /EXACT. In /FAST mode, the range specified with /ERROR

is expanded to match spacing of records in the peak index files.

For example, if the peak index records cover the ranges, 15.15-15.3,

15.3-15.45, 15.45-15.6, etc., then if

/PEAK=15.5/ERROR=0.1/FAST

is used, then the error range will be expanded to select peaks

between 15.3 and 15.6.

An error value may be specified for each peak, but if the number

of values in the error_list is less than the number of peaks in

the peak_list, the last error value will be used for all

remaining peaks. If the /ERROR parameter is not used, the

program will prompt for a single error value for all peaks.

The error value should be specified in the same units (d-space

or two-theta) as the peak positions.

The /PEAKS qualifier must precede this qualifier.

The following qualifiers may not be used with /ERROR:

/CARDS, /NAME, /ELEMENTS, /E_SPAN, /COUNT,

/OPTIONAL, or /O_SPAN.

/DSPACE

Using this qualifier overrides the current wavelength setting and forces

peaks to be input as d-spaces. This qualifier cannot be used with

/WAVELENGTH and will be ignored if used with a qualifier other than

/PEAKS.

/WAVELENGTH=wave_length

Using this qualifier overrides the current wavelength setting and forces

peaks to be input as two-theta angles at wave_lengthAngstrom. This qualifier

cannot be used with /DSPACE and will be ignored if used with a qualifier

other than /PEAKS.

/LEVEL=peak_level

Specifies a relative intensity threshold for peak positions specified

with the /PEAKS qualifier. The threshold value is used slightly

differently in /FAST mode versus /EXACT mode. In /EXACT mode,

the value specifies the exact intensity threshold (in percent relative

intensity). For example if /PEAKS=20.5/ERR=0.1/LEVEL=75/DSPACE/EXACT

is specified, entries with peaks having d-spaces from 20.4 to 20.6 Angstrom

with relative intensities of 75% or greater will be located.

In /FAST mode, the threshold value specifies a peak index to

be searched. If there is no peak index at this value, a peak index

with a lower threshold is used (at 10 % increments). Thus in the

example /PEAKS=20.5/ERR=0.1/LEVEL=75/DSPACE/FAST, the error range is

opened (see /ERROR) and the intensity threshold is also possibly

lowered, so that potentially more entries will be selected than in

/EXACT mode. It is also possible to specify /LEVEL=3s in /FAST mode.

This specifies an index of the three strongest peaks in each entry.

The /PEAKS qualifier must precede this qualifier.

The following qualifiers may not be used with /LEVEL:

/CARDS, /NAME, /ELEMENTS, /E_SPAN, /COUNT

/OPTIONAL, or /O_SPAN.

/EXACT

Use /EXACT with /ELEMENTS to specify that only entries

containing all of the specified elements and no others

match as the specified logical criterion. It is possible to widen

this criterion so that certain elements are allowed, but not required,

using the /OPTIONAL and/or /O_SPAN qualifiers.

Use /EXACT with /PEAKS to specify that the error range specified

for peak positions should not be extended, but instead be used as given.

(See the description given for /ERROR.)

The following qualifiers may not be used with /EXACT and /ELEMENTS:

/CARDS, /NAME, /PEAKS, /OR, /AND, /ERROR, /FAST,

/COUNT or /LEVEL.

The following qualifiers may not be used with /EXACT and /PEAKS:

/CARDS, /NAME, /ELEMENTS, /E_SPAN, /FAST, /COUNT

/OPTIONAL, or /O_SPAN.

/FAST

Use /FAST with /PEAKS to specify that the error range specified

for peak positions may be extended for most efficient searching.

(See the description given for /ERROR.)

The following qualifiers may not be used with /FAST:

/CARDS, /NAME, /ELEMENTS, /E_SPAN, /OPTIONAL, /EXACT,

/COUNT or /O_SPAN.

/OR

Use /OR with either /PEAKS or /ELEMENTS to specify that any entry

containing the specified element or peak is a match for the specified

logical criterion.

The following qualifiers may not be used with /OR:

/CARDS, /NAME, /AND, /COUNT, /OPTIONAL, or /O_SPAN.

Also, if /ELEMENTS and /OR are used, /EXACT cannot be used.

/AND

Use /AND with either /PEAKS or /ELEMENTS to specify that only entries

containing all of the specified elements or peaks match for the specified

logical criterion.

The following qualifiers may not be used with /AND:

/CARDS, /NAME, /OR, /COUNT, /OPTIONAL, or /O_SPAN.

Also, if /ELEMENTS and /AND are used, /EXACT cannot be used.

/OPTIONAL=optional_list

/O_SPAN=opt_range(s)

Use /OPTIONAL or /O_SPAN with /ELEMENTS to specify that only

entries containing all of the required elements specified in the

required_list (see /ELEMENTS and /E_SPAN) and optionally those

in the optional_list, but no others, as the specified logical

criterion. /O_SPAN is equivalent to /OPTIONAL except that ranges

of elements may be specified.

Examples of the use of this qualifier are:

/OPTIONAL=SI

/ig=(si,o,H)

/O_SPAN=(H,BE)

/O_SPAN=(BE,H) (equivalent to previous)

/OPTIONAL=(H,HE,LI,BE) (equivalent to previous)

/O_SPAN=(H,BE,C,F) specifies two ranges: from H to Be and

from C to F.

Note that capitalization is ignored and the elements must be surrounded

by parentheses and separated by commas, if more than one element is

specified.

Ranges may be specified in either ascending or descending order.

To specify more than one range, specify more than one pair of endpoints.

The following qualifiers may not be used with /OPTIONAL or /O_SPAN:

/CARDS, /NAME, /PEAKS, /FAST, /OR, /AND,

/ERROR, /COUNT, or /LEVEL.

Qualifier /EXACT is assumed to be present if /OPTIONAL or /O_SPAN

is used and thus is optional. /O_SPAN may be used with /OPTIONAL.

===> COMMAND  DCL

Spawns a subprocess to execute DCL commands, type LOGOUT to

return back to LOGIC without affecting the "current list".

This command is useful for quick DCL "errands" such as answering a

mail message without exiting the program.

===> COMMAND  DELETED

Some of the entries in the database are marked as "deleted" by the

editors of the database. If these entries are included in the database

(using the ENCODE program, PDF1) it is possible to select that these

entries will be included or excluded from the operations of this

program.

This command causes logical name PS_DELETED to be set so that

subsequent operations with the program and with other programs in the

Powder Suite package will use the same option. The default mode for

the program is DELETED/EXCLUDE. However, if desired the PS_DELETED

logical name may be predefined (see customization instructions) to change

the default.

---  Qualifiers

/INCLUDE

Specifies that deleted entries will be included in the logical operations.

/EXCLUDE

Specifies that deleted entries will be not be included in the logical operations.

===> COMMAND  EXIT

Type EXIT (which can be abbreviated to EX) to exit from the program.

Alternately, press Control Z (^Z) to exit.

===> COMMAND  FILE

Allows a series of commands to be input to the program from an external

file. The file should be prepared before starting the program, or using

the DCL command to invoke an editor.

---  Parameter

file_name

A single file may be specified on the command line containing the name

of the file with commands. If the file is not specified, a prompt will be

given to request a file name.

If no file extension is supplied, the default extension is .CMD

===> COMMAND  INDEX

Displays an index of the entries in the current list, with the information

for each entry summarized to a single line.

There are three possible formats:

In "names" format, the chemical composition, chemical and mineral name

for each entry in the current list are given. In "Hanawalt" format, the

most intense lines are listed with intensity and d-spacing or angle.

This list may optionally be sorted in order of increasing d-space or

angle and the most intense lines may be permuted. In "maxD" format,

the lines are listed in order of decreasing d-space (or increasing

angle). This list may be sorted and permuted, as before.

If /PRINT is not specified, the list is displayed on the terminal in

sections. After each section, a "Continue?" prompt appears. If the

response is "N", "P" or ^Z, no more sections will be displayed.

If "P" is specified, the complete list will be printed. Enter "Y"

or a blank line for the next section.

---  Qualifiers

/HANAWALT=sort_lines

/HANAWALT

The list is displayed in Hanawalt format -- an entry number followed by

the d-spaces or angles ordered by intensity.

Entries are listed by PDF file in sequential order, unless the sort_lines

option is specified, in which case the records are sorted by

increasing d-spacing or angle.

sort_lines specifies the number of peaks to be used for sorting:

/HANAWALT=0 -- the list is sorted by set and entry number.

/HANAWALT=1 -- the list is sorted based on the

most intense line.

/HANAWALT=sort_lines -- (sort_lines=2,3,4 or 5),

the sort_lines most intense lines are permuted and then sorted,

thus the length of the list increases by a factor of sort_lines.

/MAXD=sort_lines

/MAXD

The list is displayed in maxD format -- an entry number followed by

the d-spaces or angles ordered by decreasing d-space (increasing angle).

Entries are listed by PDF file in sequential order, unless the sort_lines

option is specified, in which case the records are sorted by

increasing d-spacing or angle.

sort_lines specifies the number of peaks to be used for sorting:

/MAXD=0 -- the list is sorted by set and entry number.

/MAXD=1 -- the list is sorted based on the

highest d-space line.

/MAXD=sort_lines -- (sort_lines=2,3,4 or 5),

the sort_lines highest d-space lines are permuted and then sorted,

thus the length of the list increases by a factor of sort_lines.

/NAMES

The "current list" is displayed with chemical formula, compound name and

mineral name. Entries are listed in sequential order by set and entry number.

/PRINT

Prints the index rather than displaying it on the terminal.

===> COMMAND  MATCH

Command MATCH decreases the number of entries in the current table using

a logical .AND. operation to remove all entries which do not match a

specified logical criterion. Command AND is completely equivalent to

command MATCH.

There are five possible logical criteria that may be established:

/ELEMENTS A list of entries is specified by a partial or

complete elemental composition

/CARD     A list of entries is specified by number

/PEAKS    A list of entries is specified by positions of the

major peaks in the pattern.

/NAME     A list of entries is specified by chemical/mineral name

/COUNT    A list of entries is specified by the number of different

elements in the chemical formula.

Qualifiers may be specified in only certain combinations. Square brackets

([ ]) indicate parameters that are allowed but are not required.

For example, if /ELEMENTS/OPTIONAL is used, /EXACT is assumed,

and need not be specified explicitly.

Angle brackets (< >) indicate that only one of the enclosed

parameters parameters may be chosen. For example, for /PEAKS/OR

either /DSPACE or /WAVELENGTH may be used, but not both.

Curly brackets ({ }) indicate that more than one of the the enclosed

parameters parameters may be chosen, but at least one must be chosen.

For example, either /ELEMENTS or /E_SPAN or both may be specified,

but one of the two should be specified.

MATCH {/ELEMENTS= /E_SPAN=} /OR

MATCH {/ELEMENTS= /E_SPAN=} /AND

MATCH {/ELEMENTS= /E_SPAN=} /EXACT

MATCH {/ELEMENTS= /E_SPAN=} {/OPTIONAL= /O_SPAN=} [/EXACT]

MATCH/NOELEMENTS {/OPTIONAL= /O_SPAN=} [/EXACT]

MATCH/CARDS=

MATCH/NAME=

MATCH/COUNT=

MATCH/PEAKS=/OR/FAST  [/ERROR=] [/LEVEL=] </DSPACE /WAVELENGTH=]>

MATCH/PEAKS=/AND/FAST [/ERROR=] [/LEVEL=] </DSPACE /WAVELENGTH=]>

MATCH/PEAKS=/OR/EXACT/ERROR=  [/LEVEL=] </DSPACE /WAVELENGTH=]>

MATCH/PEAKS=/AND/EXACT/ERROR= [/LEVEL=] </DSPACE /WAVELENGTH=]>

The /ELEMENTS (or /E_SPAN), /CARD, /NAME or /PEAKS qualifier

should be specified before other all other qualifiers.

---  Qualifiers

/ELEMENTS=required_list

/ELEMENTS

/NOELEMENTS

/E_SPAN=req_range(s)

Use this options to specify a list of elements which must be present for

the logical criterion to be matched. The list of elements may be used

in one of four different ways:

/OR

An entry with a composition containing any one of the

specified elements is a match.

/AND

An entry with a composition containing all of the

specified elements is a match.

/EXACT

An entry with a composition containing all of the

specified elements and no others is a match.

/OPTIONAL (and/or /O_SPAN)

An entry with a composition containing all of the

specified elements and any or none of the optional

elements, but no others no others is a match.

Use /E_SPAN to specify a range of elements which must be present for

the logical criterion to be matched. This option is equivalent to

specifying the elements individually in the required_list using

/ELEMENTS.

The /NOELEMENTS option is used if there are no required elements,

but optional elements will be specified using the /OPTIONAL or

/O_SPAN options. Thus, entries will be selected that have any

combination of the optional elements and no others. Use of this

option will allow some unexpected entries, entries with unassigned

chemical formulas. These entries may be removed using a

CULL/COUNT=0 command.

Examples of the use of these qualifiers are:

/ELEMENTS=SI

/elem=(si,o,H)

/E_SPAN=(H,BE)

/E_SPAN=(Be,H) (same as previous)

/ELEMENTS=(H,HE,LI,BE) (same as previous)

/E_SPAN=(H,BE,C,F) (specifies two ranges: from H to Be and

from C to F.

Note that capitalization is ignored and the elements must be surrounded

by parentheses and separated by commas, if more than one element is

specified. For the /E_SPAN qualifier, element ranges may be

specified in either ascending or descending order.

To specify more than one range, specify more than one pair of endpoints.

If no elements are specified with /ELEMENTS, and /E_SPAN is not

used, the program will prompt for elements.

The following qualifiers may not be used with /ELEMENTS or /E_SPAN:

/CARDS, /PEAKS, /ERROR, or /LEVEL.

The /ELEMENTS or /E_SPAN qualifier should be specified before

other all other qualifiers.

/COUNT=elm_count

/COUNT=(c_min,c_max)

/COUNT

Use this qualifier to select entries using the number of unique elements

in the chemical formula. The qualifier may be used three ways, if a

single value is specified, for example /COUNT=2, then entries are

selected with that single number of elements. If two values are used,

for example /COUNT=(2,4) the two numbers represent a range of values,

in this case 2, 3 or 4 elements are allowed. If no value is specified,

the computer will prompt for one or more values.

Note that Cr2O3 . xH2O will be counted as having 3 elements in the

formula.

No other qualifiers are allowed with /COUNT.

/CARDS=card_list

/CARDS

Use this qualifier to select entries using their ICDD-JCPDS entry number.

The entry number is always a five or six digit number consisting

of the "old" entry number plus 10,000 times the set number. For

example, entry 413 in set 7 would be coded as:

/CARD=70413 or

/CARD=070413

To specify more than one entry, separate the list with commas and

enclose the list in parentheses:

/CARD=(070413,10001)

No other qualifiers are allowed with use of the /CARDS qualifier.

/NAME=compound_name

/NAME=mineral_name

A fragment of a chemical or mineral name to be matched. The name

listed on an entry must match the compound_name fragment letter

for letter (lowercase letters will be converted to uppercase). If

spaces are to be included in the fragment, the fragment should be

enclosed in quotation marks ("). For example,

/NAME=silicon

/NAME="silicon oxide"

No other qualifiers may be used with /NAME.

/PEAKS=peak_list

/PEAKS

Use this option to select entries based on the location of the

intense peaks in the pattern. The peak positions must be input

as either two-theta angles or as d-spaces depending on the

current wavelength setting (see command WAVELENGTH). The

wavelength setting may be overridden using the /WAVELENGTH

or the /DSPACE qualifiers.

All entries with intense peaks (the intensity threshold is set by

the /LEVEL option) within some region around input peak

position will be selected. The actual window depends on the

value(s) used on the /ERROR option.  If more than one peak

is input, the peaks should be separated by  commas and enclosed

in parentheses.  The list of peaks may be used in one of two

different ways:

/OR  An entry with a composition containing any one of the

specified peaks is a match.

/AND An entry with a composition containing all of the

specified peaks is a match.

If no peaks are input, the program will prompt for a list of peaks.

The following qualifiers may not be used with /PEAKS:

/CARDS, /NAME, /ELEMENTS, /E_SPAN, /COUNT,

/OPTIONAL, or /O_SPAN.

Specify /PEAKS as the first qualifier on the line.

/ERROR=error_list

The error_list specifies an allowed range for each peak.

The range is used in one of two ways, depending on mode used by

the program. If the mode is /EXACT, the error range is used

exactly as specified. For example, If

/PEAK=15.5/ERROR=0.1/EXACT

is used, peaks at locations between 15.4 and 15.6 will be

selected. The other mode, /FAST, may be considerably more rapid

than /EXACT. In /FAST mode, the range specified with /ERROR

is expanded to match spacing of records in the peak index files.

For example, if the peak index records cover the ranges, 15.15-15.3,

15.3-15.45, 15.45-15.6, etc., then if

/PEAK=15.5/ERROR=0.1/FAST

is used, then the error range will be expanded to select peaks

between 15.3 and 15.6.

An error value may be specified for each peak, but if the number

of values in the error_list is less than the number of peaks in

the peak_list, the last error value will be used for all

remaining peaks. If the /ERROR parameter is not used, the

program will prompt for a single error value for all peaks.

The error value should be specified in the same units (d-space

or two-theta) as the peak positions.

The /PEAKS qualifier must precede this qualifier.

The following qualifiers may not be used with /ERROR:

/CARDS, /NAME, /ELEMENTS, /E_SPAN, /COUNT,

/OPTIONAL, or /O_SPAN.

/DSPACE

Using this qualifier overrides the current wavelength setting and forces

peaks to be input as d-spaces. This qualifier cannot be used with

/WAVELENGTH and will be ignored if used with a qualifier other than

/PEAKS.

/WAVELENGTH=wave_length

Using this qualifier overrides the current wavelength setting and forces

peaks to be input as two-theta angles at wave_lengthAngstrom. This qualifier

cannot be used with /DSPACE and will be ignored if used with a qualifier

other than /PEAKS.

/LEVEL=peak_level

Specifies a relative intensity threshold for peak positions specified

with the /PEAKS qualifier. The threshold value is used slightly

differently in /FAST mode versus /EXACT mode. In /EXACT mode,

the value specifies the exact intensity threshold (in percent relative

intensity). For example if /PEAKS=20.5/ERR=0.1/LEVEL=75/DSPACE/EXACT

is specified, entries with peaks having d-spaces from 20.4 to 20.6 Angstrom

with relative intensities of 75% or greater will be located.

In /FAST mode, the threshold value specifies a peak index to

be searched. If there is no peak index at this value, a peak index

with a lower threshold is used (at 10 % increments). Thus in the

example /PEAKS=20.5/ERR=0.1/LEVEL=75/DSPACE/FAST, the error range is

opened (see /ERROR) and the intensity threshold is also possibly

lowered, so that potentially more entries will be selected than in

/EXACT mode. It is also possible to specify /LEVEL=3s in /FAST mode.

This specifies an index of the three strongest peaks in each entry.

The /PEAKS qualifier must precede this qualifier.

The following qualifiers may not be used with /LEVEL:

/CARDS, /NAME, /ELEMENTS, /E_SPAN, /COUNT

/OPTIONAL, or /O_SPAN.

/EXACT

Use /EXACT with /ELEMENTS to specify that only entries

containing all of the specified elements and no others

match as the specified logical criterion. It is possible to widen

this criterion so that certain elements are allowed, but not required,

using the /OPTIONAL and/or /O_SPAN qualifiers.

Use /EXACT with /PEAKS to specify that the error range specified

for peak positions should not be extended, but instead be used as given.

(See the description given for /ERROR.)

The following qualifiers may not be used with /EXACT and /ELEMENTS:

/CARDS, /NAME, /PEAKS, /OR, /AND, /ERROR, /FAST

/LEVEL or /COUNT.

The following qualifiers may not be used with /EXACT and /PEAKS:

/CARDS, /NAME, /ELEMENTS, /E_SPAN, /FAST, /COUNT

/OPTIONAL, or /O_SPAN.

/FAST

Use /FAST with /PEAKS to specify that the error range specified

for peak positions may be extended for most efficient searching.

(See the description given for /ERROR.)

The following qualifiers may not be used with /FAST:

/CARDS, /NAME, /ELEMENTS, /E_SPAN, /OPTIONAL, /EXACT,

/COUNT or /O_SPAN.

/OR

Use /OR with either /PEAKS or /ELEMENTS to specify that any entry

containing the specified element or peak is a match for the specified

logical criterion.

The following qualifiers may not be used with /OR:

/CARDS, /NAME, /AND, /COUNT, /OPTIONAL, or /O_SPAN.

Also, if /ELEMENTS and /OR are used, /EXACT cannot be used.

/AND

Use /AND with either /PEAKS or /ELEMENTS to specify that only entries

containing all of the specified elements or peaks match for the specified

logical criterion.

The following qualifiers may not be used with /AND:

/CARDS, /NAME, /OR, /COUNT, /OPTIONAL, or /O_SPAN.

Also, if /ELEMENTS and /AND are used, /EXACT cannot be used.

/OPTIONAL=optional_list

/O_SPAN=opt_range(s)

Use /OPTIONAL or /O_SPAN with /ELEMENTS to specify that only

entries containing all of the required elements specified in the

required_list (see /ELEMENTS and /E_SPAN) and optionally those

in the optional_list, but no others, as the specified logical

criterion. /O_SPAN is equivalent to /OPTIONAL except that ranges

of elements may be specified.

Examples of the use of this qualifier are:

/OPTIONAL=SI

/ig=(si,o,H)

/O_SPAN=(H,BE)

/O_SPAN=(BE,H) (equivalent to previous)

/OPTIONAL=(H,HE,LI,BE) (equivalent to previous)

/O_SPAN=(H,BE,C,F) specifies two ranges: from H to Be and

from C to F.

Note that capitalization is ignored and the elements must be surrounded

by parentheses and separated by commas, if more than one element is

specified.

Ranges may be specified in either ascending or descending order.

To specify more than one range, specify more than one pair of endpoints.

The following qualifiers may not be used with /OPTIONAL or /O_SPAN:

/CARDS, /NAME, /PEAKS, /FAST, /OR, /AND,

/ERROR, /COUNT, or /LEVEL.

Qualifier /EXACT is assumed to be present if /OPTIONAL or /O_SPAN

is used and thus is optional. /O_SPAN may be used with /OPTIONAL.

===> COMMAND  READ

Reads a table (subfile) written by the WRITE command, prepared

using some other program, such as MATCH, or one of the standard

ICDD-JCPDS subfiles. The READ command can also be used to read a list of

entries from a free-format ASCII text file or to select a list of

entries in a range by number.

---  Parameter

file_name

A single file may be specified on the command line containing the name

of the file to be read. If the file is not specified, a prompt will be

given to request a file name to be read.

If no file extension is supplied, the default extension is .USRPDF

To read one of the standard subfiles, omit the file_name parameter

and specify the /SUBPDF qualifier.

---  Qualifiers

/SUBPDF=sub_file

Use this option to read in one of the standard subfiles, such as the

the inorganic subfile.

The available subfiles are:

FULL         all entries in the database

NULL         null table (a table without any entries)

INORGANICS   Inorganic materials

ORGANICS     Materials with a C-H bond

COMMON       commonly found materials or phases

MINERALS     Minerals, natural and synthetic

METALS       metals, alloys and related materials

NIST         patterns recorded by NIST (NBS)

FORENSIC     materials of forensic interest

ZEOLITES     zeolites and molecular sieves

DELETED      superceeded but not removed patterns (if any)

LOCAL        locally defined patterns (if any)

SUPERCON     superconductors and related materials

EXPLOSIVES   explosives and related materials

EDUCATION    educationally relevant materials

CEMENT       materials related to cement production

CORROSION    corrosion products and related materials

Note that the sub_file name may be abbreviated by only including

the initial letters as long as the name is uniquely specified.

Also, note that by default, "DELETED" patterns are not included,

unless the DELETE/INCLUDE command is used to force inclusion

of deleted patterns.

Do not include a file_name parameter if /SUBPDF is used.

/LIST=list_file

Use this option to read a list of entries from a text file, named

list_file (with default extension .CARDS). This file will usually

be prepared prior to starting LOGIC through use of a text editor or

with a non-Powder Suite program.

The list_file contains a list of ICDD-JCPDS entries with one entry

entry per line in the file. If an invalid number is used, a warning

is reported.

The entry number may entered either with or without a comma separating

the set number from the entry number (38,1 and 380001 are equivalent).

Comments may be included on each line provided that two commas or

are placed before the comment or the comment starts after column 20

(see below). Blank lines in the file are ignored. A negative set number

terminates reading the file, but use of a -1 is not required.

Sample list_file:

380001,,         this is the 1st entry in the list

390001,,         this is the last entry

-1

The /LIST option may not be used with /SUBPDF or /RANGE.

Omit the file_name parameter if /LIST is used.

/RANGE=(entry1,entry2)

Create a list with entries starting with entry1 and ending with

entry2. The number must contain both the set and entry numbers.

For example READ/RANGE=(360000-369999) will select all entries

in set 36.

The /RANGE option may not be used with /SUBPDF, /LIST.

Omit the file_name parameter if /RANGE is used.

/OR

Specify this option to merge the requested subfile with the current

table. Thus, the new current table will contain the entries in either

the subfile OR the current table. (NEW_TABLE = OLD_TABLE .or. SUB_FILE)

/OR may not be used with /AND, /NOT  or /REPLACE

/AND

Specify this option to match the requested subfile against the current

table. The new current table will contain the entries in both the subfile

AND the current table. (NEW_TABLE = OLD_TABLE .and. SUB_FILE)

/AND may not be used with /OR, /NOT or /REPLACE

/NOT

Specify this option to cull entries from the requested subfile from

the current table. All entries present in the both subfile and the current

table will be removed from in the new current table.

(NEW_TABLE = OLD_TABLE .and. .not. SUB_FILE)

/NOT may not be used with /OR, /AND or /REPLACE

/REPLACE

Specify this option to set the new current table to be the same as the

requested subfile. (NEW_TABLE = SUB_FILE)

/REPLACE may not be used with /OR, /AND or /NOT

===> COMMAND  TABLES

Prints a table of the d-spaces and chemical name for all entries in

the current list on the PS hardcopy listing device.

If the wavelength has been set (see command WAVELENGTH), both d-spaces

and two-theta angles are printed.

---  Qualifiers

/FULL

If this qualifier is used, the full ICDD-JCPDS (PDF-2) entry is displayed.

===> COMMAND  WAVELENGTH

Peaks positions may be input or output as d-spacings (in Angstrom) or

as two-theta (Bragg) angles corresponding to a given x-ray or neutron

wavelength. This command is used to select between d-spacings

and angles and to select a wavelength for the latter option.

The current mode of the program is displayed in the header line at the

top of the computer screen; the default when the program is started

is d-space mode.

---  Parameter

wave_length

A single wavelength value may be specified directly, where wave_length

is the x-ray or neutron wavelength in Angstrom. (The target_type will be

assumed to be SY for synchrotron radiation).

If 0.0 is used for the wave_length, the program will enter d-space

mode (same as specifying WAVELENGTH/NOTARGET).

If wave_length is used, /TARGET and /WTYPE should not.

---  Qualifiers

/TARGET=target_type

/TARGET

/NOTARGET

Specifies an x-ray target for conventional x-ray sources.

The choices for targets are:

Cu, Mo, Ni, Co, Fe, Cr, Ag, NU, or SY.

Specifying /TARGET without a target_type is equivalent to

/TARGET=CU. (Capitalization of the target type is ignored.)

If NU or SY (for neutron or synchrotron radiation) is used,

the program prompts for a wavelength.

For input/output using d-spacings specify /NOTARGET

The /TARGET option cannot be specified with a wave_length value.

/WTYPE=wave_type

wave_type may be either 1, 2 or 3 which determines

the exact value used for the wavelength specified with /TARGET.

The choices are:

1 to use K alpha 1 radiation

2 to use K alpha 2 radiation

3 to use the intensity weighted average of K alpha 1 and K alpha 2

radiation.

Specifying /WTYPE without a value for wave_type is equivalent

to specifying /WTYPE=1.

This qualifier is ignored if the radiation source has a single

wavelength, (e.g. /TARGET=NU or /TARGET=SY) or a

wave_length value is specified.

---  Examples

WAVELENGTH/TARGET=CU/WTYPE=3

The wavelength will be set to the average of Cu K alpha 1 and

K alpha 2.

WAVELENGTH/TARGET

The wavelength will be set to Cu K alpha 1

WAVE/NOTAR

Peaks positions, etc. will be displayed as d-spaces.

WAVE 0.0

Peaks positions, etc. will be displayed as d-spaces.

WAVE 2.5401

The wavelength will be set to 2.5401Angstrom.

WAVE

The user will be prompted to choose a wavelength or two use

d-spacings.

WAVELENGTH/TARGET=NU/WTYPE=3

The user will be prompted to enter a wavelength; the /WTYPE=3

qualifier will be ignored.

===> COMMAND  WRITE

This command writes the current list in a format suitable for use as a

"user supplied" PDF subfile for program MATCH.

---  Parameter

file_name

A single file may be specified on the command line containing the

name of the file to be written. If the file is not specified, a prompt will

be given requesting a file name to be written.

If no file extension is supplied, the default extension is .USRPDF

---  Examples

WRITE MYINDEX.FILE

This writes file MYINDEX.FILE on the current default device and

directory.

WRITE XRAY:[mother.group]myindex

This writes file [MOTHER.GROUP]MYINDEX.USRPDF on device XRAY.

WRITE

The program will prompt for the name of a file to write.

***

Logic Examples

%------------%

A few examples are presented to show how LOGIC may be used. Note that

there may be several ways to achieve the same results [as is shown in

in Example 1(a) and (b).] Also note that the READ and

WAVELENGTH commands may not be needed, as the subfile may be

selected when the program starts and the wavelegth may have already

been set appropriately.

Example 1(a):

Find all entries in the full database that contain both Si and O and may

also contain Al and/or H (or neither) The resulting entries are then

examined using the BROWSE command.

READ/REPLACE/SUBPDF=FULL\_JCPDS

MATCH/ELEMENT=(SI,O)/OPTIONAL=(AL,H)

BROWSE

Example 1(b):

Find all entries in the full database that contain both Si and O and may

also contain Al and/or H (or neither). This yields the same results as

Example 1(a).

READ/REPLACE/SUBPDF=NULL

MATCH/ELEMENT=(SI,O)/OPTIONAL=(AL,H)

BROWSE

Example 2:

Find all entries in the metals section of database that contains Fe

or Mn and two or three other elements. The entries must have a 30% or

greater intensity peak within approximately 0.25 degrees of 12, 14

or 16 degrees and may not have any strong peaks (greater than 70%)

between approximately 5 and 10 degrees. (Angles are two-theta values

for Cu K alpha 1 radiation).

Two lisings of these entries are made, one with the entry numbers and

the other with the most intense three lines permuted and sorted. The

first list will be printed and the second list will be displayed on

the screen and then optionally printed.

READ/REPLACE/SUBPDF=METAL

MATCH/ELEMENT=(FE,MN)/OR

WAVELENGTH/TARGET=CU/WTYPE=1

MATCH/PEAK=(12,14,16)/OR/ERROR=0.25/LEVEL=70/FAST

CULL/PEAK=7.5/ERROR=2.5/LEVEL=30/FAST

INDEX/NAME/PRINT

INDEX/HANAWALT=3

Example 3:

For some complex searches, it may be necessary to perform a search in

sections and save intermediate results on files and finally combine

these files. To find all the entries in the ZEOLITE database that

contain C plus all the entries in the ORGANIC database that contain

both Al and Si, it is necessary to perform each search separately.

READ/REPLACE/SUBPDF=ZEOLITE

MATCH/ELEMENT=(C)/OR

COMMENT Carbon containing Zeolites

WRITE c-zeol

READ/REPLACE/SUBPDF=ORGANIC

MATCH/ELEMENT=(AL,SI)/AND

COMMENT Alumino-silicate organics

READ/OR c-zeol

COMMENT C-containing Zeolites and Al-Si organics