POWDER SUITE DOCUMENTATION:  OVERVIEW

The Powder Suite package of computer programs is designed for interactive

and limited batch processing of powder diffraction data. The package

consists of several programs, which are briefly discussed in this chapter,

and in more detail in a chapter devoted to each program. Also discussed

here are features that are used all programs.

*** Organization of Programs

The programs used in this package all have the same type of internal

organization, modeled after the command structure of the VAX/VMS DCL

command language. Once the program is started, any appropriate

command for that program may be entered, usually in any sequence.

The format for a command is:

COMMAND[/qualifiers] [parameters]

or

COMMAND [parameters][/qualifiers]

(the [ ] symbols are used to indicate that an option need not be specified.)

If it is desired to spread a command over several lines, end all but the

last line with a dash (-).

For most commands, it is possible to achieve the same results by

specifying command with no qualifiers or parameters and the program

will prompt for important options (prompt -- meaning the computer will

request input). It is often faster to specify options on the command

line. However, some infrequently used options on certain commands are

only available on the command line.

For example, in program CMPR, a file may be plotted by typing

PLOT

this causes the program to prompt for file number(s) and some scaling

information. Typing

PLOT 1/XLOW=10/XHIGH=40

Causes file 1 to be plotted from 10 to 40 degrees. The /TITLE option,

which puts the file's title on the plot can only be requested with a

qualifier. To get a title on a plot type one must enter

PLOT/TITLE     or

PLOT 1/XLOW=10/XHIGH=40/TITLE

Programs which offer a DEFAULT command have two modes AUTO and MANUAL.

In AUTO mode the program will use default values for options wherever

possible. However, if an option is specified on a command line, the

command line specification overrides the default.

At this time, not all options within a command may be specified

on the same line as a command, but this will be changed in later

versions of the package to allow those commands to be used in batch mode.

*** Description of Programs

A brief description of the programs currently contained in the package

follows. Complete documentation of the commands available in each program

may be found in a following chapters.

****** Program LOGIC

Program LOGIC locates entries in the JCPDS-ICDD data base either from a

JCPDS-ICDD entry number or by searching the database for compounds matching

a specified chemical composition, chemical/mineral name or with peaks

in selected regions.

The program can display or print

relative intensities versus two-theta or d-spacings for the selected entries,

or plot the same data, optionally displaying an experimentally determined

powder diffraction pattern or set of peaks on the same scale.

LOGIC may also be used to examine lists of matching patterns determined

by program SEARCH. Experimentally determined peak positions may be read

from the SEARCH parameter file for graphic display along with the

matching patterns.

****** Program SEARCH

Program SEARCH is used to search the JCPDS-ICDD database to identify

unknown phases in patterns. Peaks may be input directly to

SEARCH, or can be read from a data file written by MANIP.

Chemistry requirements may be used to limit the number of entries

that must be examined by using the LOGIC program, which may be

called from within SEARCH.

Use of chemistry also reduces the number of spurious matching

patterns by eliminate from consideration materials with

unlikely compositions.

Depending on the number of entries to be examined, the

database search may require anywhere from seconds to several minutes

of computer time, so the user has been given the option to execute the

search either interactively, or with a batch job -- freeing the terminal

for other uses.

SEARCH writes a file of parameters which may be reread and

modified in a later SEARCH run.

****** Program CMPR

Program CMPR is used for comparison of diffractograms. Data may

be plotted, added, subtracted, and/or compared to JCPDS-ICDD entries.

This program is still under development.

****** Program MANIP

Program MANIP is used for processing of a single

diffractogram. Operations that may be performed include

baseline assignment and stripping, K alpha 2 stripping,

interpolation, smoothing and peak processing.

This program is still under development.

****** Program ENCODE

Program ENCODE is actually a sequence of four programs which create the

files needed by Powder Suite from the full (PDF-2) JCPDS-ICDD database.

The first program, PDF1, reads the PDF-2 database and creates a condensed

subset of the original database called the PDF-1 database. These

entries may also be indexed by the PDF1 program and will be accessible

by the other Powder Suite programs. Optionally users may add their own

PDF-2 entries to the database using this program.

The second program, ADDPDF1, is used to add PDF-1 format entries to the

PDF-1 files written by program PDF1. These entries are easier to prepare

than PDF-2 entries. Use of this program is optional.

The third program, WRDI, is used to create the d-space index files

which are used for rapid searching of the database by peak position.

The fourth program, WRCHEM, is used to create the elemental composition

index files which are used for rapid searching of the database by

chemistry.

*** DCL Command Procedure

The VAX/VMS operating system provides a subdirectory structure

that allows users to catalog their datafiles by storing them

in different subdirectories. While fully-specified file names may be

input for file names to the Powder Suite programs, it is usually most

convienient to use the name without a device, directory and extension.

In this case, most files will be written in the current default directory

with an extension selected by the program. (Certain temporary files

will be written in a designated scratch directory, PS_TMP:).

It is suggested that users of these programs periodically "clean-up"

(delete unneeded files from) their working and scratch directories.

The clean-up process is easier if several subdirectories and

descriptive file names are used so that junk may be discerned from

still-needed information. If the user creates a subdirectory for

each project and sets current default (using the DCL SET DEFAULT

command, or the equivalent) to the appropriate subdirectory before

using the PS package, all files for that project will be placed in

that subdirectory.

To aid in using the Powder Suite package from a directory or disk

other than where the programs are stored, a command procedure named

PS.COM defines the required logical names and symbols. Typing

$ @dev:[dir.PS]PS

where dev:[dir.PS] is the location of the Powder Suite files,

starts a menu where a user can access the programs or get help

information. After the first time the procedure is called,

a symbol PS is defined which is equivalent to the above command,

so typing PS will also start the package.

For convenience the symbol PS may be defined as

$ PS    :==   @dev:[dir]PS

in the user's or system login file and the user never needs to use the

initial @dev:[dir.PS]PS command. (An alternative to this is to place

the command

$ @dev:[dir]PS INIT num

in the user's LOGIN.COM file; this defines a terminal type as num,

(see the Customization Instructions).

To run a specific command, it is possible to bypass the menu, by typing

$ PS command

for example, PS LOGIC or even PS L to start program LOGIC.

Note that program names can be abbreviated down to

any unique abbreviation (eg. C for CMPR).

The first time the PS.COM procedure is called, a second file,

PS_INIT.COM, is also executed which defines all of the symbols

and logical names required by the package. When the package is

first installed, the PS_INIT.COM should be updated to reflect

the local computer environment (device names etc.)

In addition each user can have one or more customized PS_LOCALINIT.COM

files.

The following locations are searched (in the order specified)

for the PS_LOCALINIT.COM file:

(1) the current default directory

(2) the current login directory (as defined by SYS$LOGIN).

In this way, by placing PS_LOCALINIT.COM files in different

locations, differing sets of logical names may be used by

different users or even for different projects.

The package may be reinitialized at any time by typing

PS INIT

The symbols and logical names defined in the PS_INIT.COM file

are discussed in more detail

in ASCII file CUSTOMIZE.DOC, which is distributed with the package.

The installation procedure can be used to create the

PS.COM and PS_INIT.COM files with definitions suitable for the system

where the programs have been installed.

This is discussed in more detail

in ASCII file 1README.1ST, which is distributed with the package.

*** On-line Help Information

The programs documented here all have VAX help files which may be

accessed either from inside the program or directly from the PS.COM

procedure using the HELP option. The help information is quite similar

to the text of this manual.

From inside a program, type the HELP command

to access the help menu. The options for which further information is

available will be listed following help text. For example, inside program

LOGIC, at the LOGIC> prompt, type HELP to access the help file. A list

of commands will be displayed. Then type (for example) MATCH for information

on the MATCH command. There are several sub-options within MATCH, which can

then be examined. To return to the program press the control key and the

Z key at the same time (this action is usually called a control-Z

sequence and is will be abbreviated as ^Z).

From the PS.COM procedure, first type HELP and then ask for the

specific help library by typing, for example, @LOGIC. From this point,

help is available as described above.